AASHTO STANDING COMMITTEE ON RESEARCH

AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS

NCHRP Problem Statement Outline

I.	PROBLEM NUMBER - To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Reducing Traffic Impacts When Performing Bridge Maintenance and Preservation Actions

III.	PROBLEM STATEMENT

With the continued increase in travel demands on our roads and bridges, coupled with aging and deteriorating infrastructure, highway agencies and bridge owners are faced with a significant challenge in mitigating vehicular traffic impacts during planned and unplanned bridge maintenance and preservation projects. In some instances, agencies choose to defer warranted maintenance and preservation activities (that can be accomplished using conventional methods) due to the potential impacts to traffic on the highway facility.

IV.	LITERATURE SEARCH SUMMARY

A literature search on minimizing delays in executing preservation and maintenance actions on highway structures, particularly on high volume roadways, using innovative materials, techniques, and  equipment.

Information is available on the use of rapid deployment bridge components, such as precast deck panels, and pre-stressed concrete piers, however, repairs and preservation activities are not included.

V.	RESEARCH OBJECTIVE

The purpose of this research is to document successful bridge maintenance and preservation techniques that are accomplished safely with minimum impact to traffic. For example, there are mobile bridge painting techniques that include simple scaffolding, containment systems, and paint equipment that can be rapidly deployed and used for painting overpass highway bridges. The equipment is confined to one lane of traffic and eliminates the need for rigging or other complex temporary access staging. Another example is the use of polymer-based material, pumped and gravity fed, to repair potholes on decks.  The use of innovate devices for use in traffic operations allows maintenance crews to set and remove work zones quickly and work safely on high volume roadways.  High performance materials and construction sequencing techniques allow work to be done with less interruption to traffic.

The information collected through this synthesis will be useful to bridge owners contemplating the selection of viable accelerated maintenance and preservation methods on their bridges while minimizing the impact on the traveling public

The project objectives are: (1) to locate and assemble information on successful and potentially viable products and materials; (2) to learn what practice has been used for solving or alleviating the problems; (3) to identify all ongoing research; (4) to learn what problems remain largely unsolved; and (5) to organize, evaluate, and document the useful information that is acquired.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

Recommended Funding: $80,000

Research Period:  1 year

VII.	URGENCY, PAYOFF POTENTIAL, AND IMPLEMENTATION

The ability to “get-in and get-out” quickly and still do quality work is paramount to addressing the needs of highways structures across the nation.  Deferred maintenance of infrastructure to avoid traffic impacts leads to higher cost repairs or replacement.  Effective techniques to improve efficiency of bridge repairs can result in agencies performing early intervention while repair costs are low as well as reduce user costs.

Documentation of successful practices to minimize traffic impacts in use by bridge owners in performing bridge repair and maintenance actions will assist all owner agencies in meeting the needs of in-place highway structures.

VIII.	PERSON(S) DEVELOPING THE PROBLEM

The Bridge Technical Working Group (BTWG) of the AASHTO Sub-Committee on Maintenance (SCOM)

IX.	PROBLEM MONITOR

Anwar Ahmad
Bridge Preservation Engineer
Office of Infrastructure, FHWA
1200 New Jersey Ave, SE
RM E73-414
Washington, DC 20590
202-366-8501
FAX: 
Email: anwar.ahmad@dot.gov

X.	DATE AND SUBMITTED BY

August 15, 2012 by the BTWG of the SCOM

Please submit completed problem statement to the following e-mail address:

nchrp@nas.edu

Questions on the process can be directed to the same address or cjencks@nas.edu.