PROPOSED NCHRP 20-07 PROJECT

PRODUCE CANDE-2014

Submitted by the Subcommittee on Bridges and Structures

BACKGROUND

CANDE, an acronym derived from Culvert ANalysis and DEsign, is a finite element program dedicated to the structural design and analysis of buried culverts applicable to all sizes, shapes and material types, including corrugated metal, reinforced concrete and plastics. First introduced in 1976 under the sponsorship of the FHWA, CANDE is today’s tool of choice due to the trustworthiness of the program earned over 35 years of testing and improvement.  One improvement, NCHRP Project 15-28: Modernize and Upgrade CANDE for Analysis and Design of Buried Structures, produced CANDE-2007, which included new modeling capabilities (such as large deformation analysis, LRFD design methodology, and multiple culvert systems), compatibility with Windows 7/64 bit systems, and the addition of a Graphical User Interface (GUI) for pre- and post-processing data.

The next iteration, CANDE-2013, came about with the addition of industry developed modeling capabilities, freely available on the CANDE website (http:// www.CANDEforCulverts.com), and including:

- Conrib pipe type.  This is a new pipe type with modeling capabilities for rib-shaped reinforced/concrete cross-sections. Also, the concrete stress-strain model has the ability to model fiber reinforced concrete.

- Contube pipe type.  This is another new pipe type that permits modeling circular-shaped concrete cross sections encased in fiber-reinforced plastic tubes spaced at uniform distances. Useful for new soil-bridge systems and piles in conformance with AASHTO LRFD Guide Specs.

- Link elements with death option. Links are useful for connecting one beam-element group to another in either a pinned connection or fixed-moment connection. Also, the link element offers a death option that allows simulating strut removal, culvert erosion, and creation of soil voids.

- Deeply corrugated steel structures. CANDE’s corrugated steel pipe type now includes the new AASHTO combined moment-thrust design criterion and new global buckling equation for deeply corrugated steel structures. Also, the plastic-penetration algorithm has been improved.

- Variable plastic profile properties. The input data defining plastic profile section properties has been expanded to allow variable profile geometry around the pipe’s periphery. This is useful for arch-shaped storm-water chambers that vary the plastic profile geometry from top to bottom.

Feedback from CANDE-2013 users has been very positive on these new capabilities; however, they universally expressed the need to incorporate new capabilities with the GUI.

OBJECTIVES

1.  Produce CANDE-2014, including the industry-sponsored modeling capabilities listed above that is fully integrated with the Graphical User Interface.

2.  Incorporate additional improvements in the CANDE engine and GUI as prioritized by the user community and in conformance latest AASHTO LRFD specifications.

RESEARCH PLAN

The NCHRP is seeking the insights of proposers on how best to achieve the research objective. Proposers are expected to describe research plans that can realistically be accomplished within the constraints of available funds and contract time. Proposals must present the proposers' current thinking described in sufficient detail to demonstrate their understanding of the issues and the soundness of their approach in meeting the research objective. The work proposed must be divided into tasks and proposers must describe the work proposed in each task in detail.  The research plan should build in appropriate deliverables that include the CANDE-2014 program update and supporting manuals.

The following topics may be addressed in your proposal:

- A survey to determine user community improvement priorities

- CANDE-2014 program development may include planned GUI screen views and new modeling capabilities

- Development of CANDE-2014 user manuals

- Beta testing plan

- Dissemination plan

FUNDING: $100,000

DURATION: 12 Months

Contact:

Louis N. Triandafilou, P.E.

Team Leader - Bridge and Foundation Engineering Team

Federal Highway Administration

Phone:  202.493.3059

Fax:      202.493.3477

Email:  lou.triandafilou@dot.gov