## NCHRP 20-7

## Proposed Research Needs Statement

## Subcommittee on Materials

Subcommittee on Materials, Technical Section 3b

Moe Jamshidi, Chair

## March 21, 2011

# TITLE

Evaluating Cracking Risk for Repair Materials: A Test that Considers the Combined Effects of Shrinkage and Thermal Effects

BACKGROUND / NEEDS STATEMENT

Concrete repair materials are being used in ever-increasing quantities.  According to “Vision 2020: A Vision for the Concrete Repair, Protection and Strengthening Industry”, in the U.S. alone, the annual cost to owners for repair, protection and strengthening is estimated to be between $18 billion and $21 billion.  Furthermore, estimates of the failure rate of repairs, such as those provided by the U.S. Army Corps of Engineers, are on the order of 50 %.  Clearly, this situation must be improved.  In addition to conventional concrete, specialty repair materials are often employed in order to quickly develop strength and avoid excessive delays.  Since rapid strength development is often accompanied by the generation of substantial heat, high early stiffness, and increased internal stresses, early-age cracking is a constant problem for this class of materials.  Additionally, since the repair material should bond well to the existing substrate, a high degree of restraint exists, to further exacerbate the cracking potential.

Early-age cracking susceptibility is typically evaluated using the ASTM C1581-09 restrained ring shrinkage test.  In this test, a concentric concrete or mortar shell is cast around a rigid steel ring and performance is characterized both by the levels of measured strains in the ring and the time of observed cracking of the concrete, when the concrete is exposed to a drying environment.  Unfortunately, the current ASTM test only provides restraint against samples that shrink and cannot be used to characterize materials undergoing expansion.  Further, the conventional restrained ring test does not apply in cases in which the material undergoes large variations in temperature, since the restraint changes dimension under heating and cooling.  Recently, Purdue University and NIST have developed a new dual ring temperature-controlled version of the test setup that overcomes these limitations.  In the new test, the concrete is enclosed in two concentric rings, so that both expansion and shrinkage are restrained and can be quantified.  The restraining rings are constructed of a metal with a very low thermal expansion coefficient so that the level of restraint remains nearly constant with changing temperature.  Finally, the test setup is placed in a well insulated chamber that includes a set of heating/cooling coils, so that in addition to semi-adiabatic operation, any user-selected temperature profile of interest can be imposed on the specimen.  This new test may prove particularly valuable for evaluating repair materials that often rely on expansion for improving their bonding to the existing concrete and often undergo considerable temperature changes during their first few days of curing.

RESEARCH OBJECTIVE

To evaluate the application of a dual-ring temperature controlled shrinkage/cracking test for assessing the cracking propensity of repair materials.

# WORK TASKS

Tasks anticipated in this project include the following:

- In cooperation with the AASHTO Materials Committee, identify and procure repair materials representing those most relevant to AASHTO interests

- Preparation and evaluation of dual ring specimens of the obtained repair materials

- Analysis of the results to determine applicability of new test method for evaluating cracking susceptibility of repair materials

- Drafting of an AASHTO standard test method for the new dual ring temperature-controlled technique

- Preparation of final report and submission to AASHTO Materials Committee

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $95,000.

### CONTACT PERSON

Haleh Azari, Ph.D.

Manager

AASHTO Advanced Pavement Research Laboratory (AAPRL)

National Institute of Standards and Technology (NIST)

100 Bureau Drive; Stop 8617

Building 236, Room 124

Gaithersburg, MD 20899-8617

Mr. Moe Jamshidi

Managing Engineer (Materials)

Department of Roads

P.O. Box 94759

Phone: 402-479-4750

Email: Moe.jamshidi@Nebraska.gov

Schlitter, J.L., Senter, A., Bentz, D.P., Nantung, T., and Weiss, W.J., “A Dual Concentric Ring Test for Evaluating Residual Stress Development Due to Restrained Volume Change,” Journal of ASTM International, 2010.