## NCHRP 20-7

## Proposed Research Needs Statement

Subcommittee on Bridges and Structures, T-13 Culverts

Ray M. Trujillo, P.E., NMDOT

Chair - T-13 Culverts

8/20/2015

# TITLE

Structural Design Methodology for Spray Applied Pipe Liners in Gravity Storm Water Conveyance Conduits

BACKGROUND / NEEDS STATEMENT

Spray applied pipe liners is a trenchless technology that provides a method to structurally rehabilitate concrete and metal gravity storm water conveyance conduits with minimal impact to the travelling public.  The liner consists of a cementitious or resin based material that is applied in an existing host storm water conveyance conduit via a centrifugal remote applicator or by manual application.  Sufficient soil support and stabilization of the host conduit is required prior to placement of the spray applied liner.  The liner will provide the structural load carrying capacity without the requirement to adhere to the host conduit.

No single structural design methodology for a spray applied pipe liner currently exists.  Vendors either apply design equations outlined in ASTM F1216, which are for Cured In Place Pipe (CIPP), or they use a variety of design approaches and conservatively select the maximum pipe liner material thickness.

The AASHTO Subcommittee on Bridges and Structures, T-13 Culverts and the NTPEP Technical Committee on Pipe Liners collectively agree that the technology would be well accepted and utilized; provided the spray applied pipe liner creates a fully structural solution. However, the committees have identified the lack of a nationally accepted structural design standard as a roadblock to implementation.

RESEARCH OBJECTIVE

Recommend a structural design methodology for both cementitious and resin based pipe liners for gravity storm water conveyance conduits.  Methodology will be used to develop an AASHTO design specification.

Recommend an accelerated methodology to determine the minimum conduit service life of 50 years.  Methodology will be utilized by NTPEP Technical Committee on Pipe Liners work plan for spray applied liners.

# WORK TASKS

Tasks anticipated in this project include the following:

- Review vendor suggested structural design methodologies for cementitious and resin based pipe liners.

- Review completed and active research that pertains to spray applied pipe liners.

- Survey the State DOT’s and Canadian Agencies to identify use and inspect several installations

- Recommend a structural design methodology for cementious and resin based spray applied liners that includes:

  - LRFD Live and Dead Loads

  - Host Conduit Conditions and Site Parameters

  - Pipe Liner Material Properties

  - Identify material testing requirements

- Recommend a minimum pipe liner material thickness for:

  - cementious spray applied liner

  - resin based spray applied liner.

# URGENCY

Several DOT’s have implemented spray applied pipe liners despite the lack of a national design standard or with the understanding that they will be designed according to CIPP design equations.  Other DOT’s are waiting for a national design standard to be developed prior to use of the technology.

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $100,000.

### CONTACT PERSON

Ray M. Trujillo, P.E.

Subcommittee Chair - T-13 Culverts

Bridge Bureau Chief, New Mexico Department of Transportation

1120 Cerrillos Rd Santa Fe, NM 87504

Phone: 505-827-5448

Fax: 505-827-5345

Raymond.Trujillo@state.nm.us

Karen Byram

Technical Committee Chair- Spray Applied Pipe Liner NTPEP

State Product Evaluation Administrator, Florida DOT

605 Suwannee Street, MS75

Tallahassee, Florida 32399-0450

850-414-4353

Karen.byram@dot.state.fl.us

Jeffrey E. Syar, P.E.

Technical Committee Vice Chair- Spray Applied Pipe Liner NTPEP

Administrator, Office of Hydraulic Engineering, Ohio DOT

1980 West Broad Street

Columbus, Ohio 43223

614-275-1373

Jeffrey.syar@dot.state.oh.us