## NCHRP 20-7

## Proposed Research Needs Statement

## Subcommittee on Right of Way, Utilities, and Outdoor Advertising Control

## July 28, 2014

# TITLE

Best Practices: Horizontal directional drilling within highway rights of way

BACKGROUND / NEEDS STATEMENT

State DOTs are increasingly receiving proposals by utilities to install underground lines within highway rights of way using horizontal directional drilling (HDD) techniques.  HDD is a trenchless construction method that involves guiding a drill bit along a prescribed arc and subsequently pulling a casing pipe back through the drilled alignment.  Drilling is conducted using drilling fluid (typically a mixture of water and bentonite) that is continuously pumped to the drill bit to facilitate the removal of cuttings, stabilize the bore hole, and cool the cutting head.  Pipes can be made of materials such as PVC, polyethylene, polypropylene, ductile iron, and steel.

HDD has many notable benefits when compared to other construction methods, including minimal surface disturbance, versatility, the ability to install relatively large diameter lines at significant depths, and shorter completion times; however, the practice has also raised a number of questions related to geotechnical concerns, the ability to collect accurate as-built data and/or locate the lines, appropriate materials requirements, and whether special provisions to manage risk should be incorporated into documents that permit HDD.

RESEARCH OBJECTIVE

Provide guidance on issues and best practices for the accommodation of underground utility lines constructed using HDD technologies.

# WORK TASKS

Tasks anticipated in this project include the following areas related to HDD practices:

- Identify and compile industry standards and best practices related to:

  - Mitigation of ground settlement;

  - Standards for pipe material types relative to depth and/or geotechnical characteristics – and whether uncased lines can be installed under certain circumstances;

  - Development of as-built data and planning for post-construction locating.

- Survey all states and follow up as necessary to identify and compile best practices and policies related to the following areas:

  - Triggers for geotechnical subject matter review (outside of utilities offices);

  - Construction oversight;

  - Special provisions related to risk management (insurance and indemnification);

  - Monitoring requirements (settlement and damage to highway facilities);

  - Requirements for as-built plans and location technologies.

- Identify issues that need further attention, if any.

- Write and publish a best practices guide as a reference for developing state requirements and policies related to HDD.

# URGENCY

As HDD practices become more common, it will be important for states to better understand pertinent issues and practices for accommodation.  This information will be valuable to decision makers in protecting the integrity of highway infrastructure, avoiding utility conflicts, and ensuring that states are exercising appropriate diligence when considering authorization of HDD within highway rights of way.

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $100,000.

### CONTACT PERSON

Ahmer Nizam

nizama@wsdot.wa.gov