AASHTO STANDING COMMITTEE ON RESEARCH

AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS

20-7 Problem Statement Outline

I.	PROBLEM NUMBER

To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Maintenance Actions for Fatigue Cracking in Steel Bridge Structures.

III.	STATEMENT OF THE RESEARCH PROBLEM

As of December 2013, there were 607,751 bridges in the U.S. national bridge inventory, and 181,095 of those bridges have steel superstructures.   Fatigue-induced cracking in steel bridge members is a common and long-standing problem in our structure maintenance program.  If a fatigue crack is allowed to grow and reach a critical length, fracture may occur with the possibility of structural failure of the member.  Fatigue cracks may be caused by both in-plane and out-of-plane effects.

The vulnerability of the steel bridge superstructure to further damage depends on the loading conditions and crack locations. As the whole bridge flexes and deflects more with crack growth, the load capacity decreases. Immediate replacement of a bridge or superstructure components with fatigue cracks is often not possible due to funding priorities, access, traffic, and environmental issues.

Substantial research effort has previously been devoted to understanding the sources of fatigue-prone details and to developing engineering solutions to prevent such cracking through enhanced design and construction practices.  However, a substantial portion of the existing bridge infrastructure comprises steel structures and systems that contain fatigue-cracked or fatigue-prone elements.

Bridge Preservation and Bridge Maintenance Engineers and Practitioners require a current summary of transportation agency and industry practices and procedures used to repair or mitigate fatigue cracks in existing bridges, and the effectiveness of the repair methods.

This project would support the performance management and research strategic focus areas of the AASHTO Subcommittee on Maintenance Bridge Technical Working Group.

IV.	LITERATURE SEARCH SUMMARY

The following is a list of research projects that were previously conducted under NCHRP relating to fatigue. The last one, which culminated in NCHRP report 721, provides guidance on analysis and design practices related to fatigue, but does not address methods to mitigate fatigue issues in existing structures.

- NCHRP 12‐15(5) ‐ Fatigue Behavior of Variable Loaded Bridge Details Near the Fatigue Limit

- NCHRP 12‐28(03) ‐ Fatigue Evaluation Procedures for Steel Bridges

- NCHRP 12‐28(06) ‐ Distortion‐Induced Fatigue Cracking in Steel Bridges

- NCHRP 12‐30 ‐ Fatigue of Cables in Cable‐Stayed Bridges

- NCHRP 12‐40 ‐ Fatigue Criteria for Modular Bridge Expansion Joints

- NCHRP 12‐81 ‐ Evaluation of Fatigue on the Serviceability of Highway Bridges

Also related would be the 2005 NCHRP Synthesis 354 – Inspection and Management of Bridges with Fracture‐Critical Details and the FHWA Manual for Repair and Retrofit of Fatigue Cracks in Steel Bridges.

The FHWA Manual for Repair and Retrofit of Fatigue Cracks in Steel Bridges was based on a workshop conducted in 2002, the data was originally compiled in 2005, and the manual was published in 2013.  This publication provides summaries of repair and retrofit procedures; however, the publication does not appear to include an assessment of effective strategies currently used by transportation agencies and industry.

V.	RESEARCH OBJECTIVE

The objective of this project is to provide the information needed to develop guidelines to assist bridge owners in selecting the most appropriate strategies, procedures, and details for repairing fatigue cracks in steel bridges.  The guidelines should be presented in a format suitable for consideration as a guide specification by the AASHTO Highway Subcommittees on Maintenance and Bridges and Structures.

This project would focus on gathering and summarizing current effective strategies, procedures, and details for crack repairs and any information on the known performance of repair strategies.

This project would provide the information needed to develop guidelines to support maintenance and preservation engineering decisions to deal with fatigue cracks in steel bridge members. The guidelines would also provide practical procedures and guidance to agency field forces and contractors performing repair work.

A current assessment of transportation agencies and other industries is needed, based on current effective practices, for the following:

- Effective repair strategies that restore or improve the cracked members and that retard the rate of future cracking, including any new emerging technologies such as cold hole expansion.

- Example plan details and specifications used by transportation agencies and industry for the repair and retrofit of fatigue cracks.

- Current data regarding the effectiveness of repairs. The survey should include  repair  of fatigue cracks caused by both in-plane and out-of-plane effects.

The development of guidelines would provide a concise reference (and provide the framework for an AASHTO guide specification) on effective techniques used to address fatigue cracks and has the potential to significantly reduce ongoing maintenance costs to owning agencies of such structures. Further, identification and broad implementation of such effective practices could reduce the number of such structures that are deemed unsustainable and potentially save significant costs for unnecessary bridge/superstructure replacements.  There could be additional indirect benefits in terms of user costs by reducing impact of maintenance/construction related disruptions to traffic flow.

The work for this study should be divided into tasks, and the research plan should describe the work proposed in each task in detail. The tasks must be divided into two phases. Phase I will consist of information gathering and planning tasks, culminating in the submittal of an interim report. The interim report will describe the work completed in the Phase I tasks and provide an updated work plan for the Phase II tasks. The updated Phase II work plan should address the manner in which the information obtained in Phase I will be used to satisfy the research objective.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

Recommended Funding:

$100,000

Research Period:

12 months

VII.	PERSON(S) DEVELOPING THE PROBLEM

Jeff Milton, Chair – Bridge Technical Working Group of the AASHTO Subcommittee on Maintenance

Bridge Preservation Specialist - Structure and Bridge Division 
Virginia Department of Transportation

4219 Campbell Avenue

Lynchburg, VA  24501
(434) 856-8278 
(434) 841-1463 
Jeffrey.Milton@VDOT.Virginia.gov

This topic was identified as the top priority of the TRB AHD30 Structures Maintenance

Committee in 2013, and has also received general support from TRB committees AFF10, and AFF20.

This problem statement  is endorsed by the Bridge Technical Working Group (BTWG) of AASHTO Sub-Committee on Maintenance (SCOM).

This problem statement is also endorsed by Bruce Johnson, Chair of Committee T-9 of the AASHTO Subcommittee on Bridges and Structures.

VIII.	PROBLEM MONITOR

Jeff Milton – Chair, Bridge Technical Working Group of the AASHTO Subcommittee on Maintenance

Bridge Preservation Specialist – Structure and Bridge Division

Virginia Department of Transportation

4219 Campbell Avenue

Lynchburg, VA  24501

Phone:  (434) 856-8278

Cell:  (434) 841-1463

E-mail:  Jeffrey.Milton@VDOT.Virginia.gov

This problem statement is endorsed by the Bridge Technical Working Group (BTWG) of the AASHTO Subcommittee on Maintenance (SCOM).

IX.	DATE AND SUBMITTED BY

September 12, 2014 by the Bridge Technical Working Group (BTWG) of the AASHTO Subcommittee on Maintenance (SCOM).

-----------------------------------------------------------------------

Please submit completed problem statement to the following e-mail address:

nchrp@nas.edu

Questions on the process can be directed to the same address or cjencks@nas.edu.