NCHRP 20-7

Proposed Research Needs Statement

Washington State Department of Transportation (WSDOT)

Jeff Carpenter, P.E.

August 25, 2017

site-calibrated sediment transport model for gravel-bed rivers

SUMMARY

We propose the development of a software package that models sediment transport of gravel-bed rivers using up-to-date, state of science sediment transport equations accompanied by practical guidance and instruction of software use. Particularly in dynamic settings, the proposed software model and guidance will improve engineering design, hazard assessment, and maintenance strategies associated with Department of Transportation (DOT) infrastructure in or adjacent to gravel-bed rivers. This is a two-phased project. A beta version of the software model would be completed within a 12-month period in Phase 1 for an estimated cost of $99,995. Phase 2 involves the development of case studies, guidance manual, and training materials, as well as the publication of a Version 1.0 of the software for an estimated cost of $45,000. Phase 2 would be funded by state and potentially federal partners. The intended users are DOT engineers and geomorphologists, environmental consultants contracted by DOTs, and other public agencies, private and non-profit entities with similar analytical needs. The proposed work is applicable to DOTs across the United States (US) wherever gravel-bed rivers are found.

LETTERS OF SUPPORT

California Department of Transportation (Bruce Swanger)

Oregon State Department of Transportation (Michael Bufalino)

US Forest Service (Dan Cenderelli)

US Geological Survey (Rose Wallick)

BACKGROUND

Across the United States, DOTs regularly conduct business in or adjacent to gravel-bed rivers. In gravel-bed rivers, sediment transport is the primary driver of sudden and/or long-term channel change, and determines how channels respond to engineered modifications associated with DOT infrastructure.  In many cases, accurate estimation of sediment transport is the critical factor for successfully designing a project that resists damage, requires less maintenance, and/or maintains the quality of instream habitat.

Recent advances in the science of sediment transport have narrowed the uncertainties of model input, such as shear stress distribution, streambed grain-size distribution, and the influence of grain-size distribution on initiation of motion. Uncertainties from surface structures such as patchiness and clusters persist, but modeling advances now include methods to account for this complexity by use of site calibration. These advances represent what we would like to bring to the standard of practice. Specifically, we propose to develop new, scientifically up-to-date, field-verified modeling software and practical guidance on when and how to employ sediment transport modeling in gravel-bed streams.  Such guidance and modeling software would inform:

- Stream Simulation and Morphologic Simulation culvert design for fish passage and geomorphic stability, particularly in dynamic settings such as alluvial fans and transitions of channel slope or confinement;

- Design criteria for bridge and roadway projects at risk from channel migration and spatially-extensive bed scour or gravel accumulation;

- Hazard assessment of DOT infrastructure in or near rivers potentially subject to sediment flux originating from dam failure or removal, wildfires, landslides, volcanic eruptions, debris avalanches, and valleys destabilized by glacier retreat as the result of climate change;

- Design of bank stabilization projects in a manner that maintains integrity of habitat and adjacent streambanks; and,

- Maintenance strategies for DOT infrastructure in rivers and streams, such as reconfiguring culverts on small creeks to prevent blockage from heavy aggradation that requires annual dredging.

DOTs may not pursue sediment transport modeling in river engineering projects because of a long-standing belief that this type of analysis is costly, inaccurate, and difficult to interpret.  This perception underscores the need for improved practical guidance that addresses when sediment transport modeling is appropriate, and does so in a manner that is scientifically up-to-date.  DOTs need up-to-date methods for safely and accurately gathering field data, instruction on how to perform the modeling and interpret the supporting information.

Traditionally, transportation engineers use threshold of motion analysis for designing a static streambed to resist erosion during extreme flood events in order to protect transportation infrastructure.  However, this approach often results in very poor aquatic habitat due to an overly-hardened and immobile streambed.  If a stated project goal is to mimic a natural streambed for aquatic organism passage and to ensure continuity of geomorphic processes, specifically sediment transport, then a more sophisticated approach is needed for design of a mobile alluvial streambed. Where appropriate, a more nuanced level of modeling should take into account the structure of the streambed and its interaction with the sediment in transport.

Site-Calibrated Sediment Transport (SCST) Model

The site-calibrated sediment transport (SCST) model is one of the most effective ways to conduct sediment transport analysis on an alluvial streambed.  Site-calibrated models apply one or more bedload measurements to adjust one or more of the model parameters derived from the literature, including threshold of motion parameters.  Equations calibrated from flumes, or equations developed from rivers that differ from the project site in size, valley type, geologic setting, or morphology, will always suffer from the suite of errors discussed in Wilcock et al. (2009).

In contrast, a site-calibrated sediment transport model provides locally calibrated values for threshold of motion for the natural streambed.  By comparing how values for threshold of motion change with the project in place versus without, it is possible to predict erosion, scour and/or aggradation.  The values of critical shear stress obtained from a calibrated model are more accurate than generalized values obtained from the literature for alluvial stream channels, and more appropriate than values from engineering handbooks designed to maintain a static streambed.

Publically available models, such as HEC-RAS and even BAGS (Bedload Assessment for Gravel-bed Streams, Pitlick et al., 2009), are insufficient.  Although BAGS has the capability of developing a site-calibrated model, it is limited in spatial extent to a single cross-section, limited in the way it computes site hydraulics, and the method used to optimize the model for minimum error.  HEC-RAS does not have the capability of doing an automated site-calibrated model, and does not include some of the appropriate models for gravel-bed rivers.

The equations proposed for inclusion in the SCST software have been successfully applied in gravel-bed rivers of the Western US.  The proposed SCST software would include a suite of algorithms for greatly enhancing accuracy of modeling results via calibration using limited bedload data collected in the field (Bakke et al., 1999; Bakke et al, 2017). This site-calibrated approach has been demonstrated to be the most cost-effective way to gain prediction accuracy, yet has been slow to gain acceptance due to not being readily available in publically available software.

The proposed research products would provide systematic, consistent guidance and methods for modeling sediment transport in gravel-bed rivers where such investigations add value to risk assessment and project design. Currently, DOTs receive work products that are inconsistent in accuracy and appropriateness to the study objectives, in part because accurate modeling of sediment transport is a field that lacks firm standards of practice. In the absence of practical technical guidance and up-to-date modeling tools, practitioners are forced to conduct sediment transport investigations using models that may be poorly adapted to local conditions and/or are out of step with advances in the science. The proposed software model and user manual would provide a suite of tools that can be consistently applied at varying levels of thoroughness and accuracy depending on the needs of the project.

The intended users of these tools are consultants contracted by DOT, DOT engineers and geomorphologists, and other public agencies, private and non-profit entities with similar analytical needs. The proposed work is applicable to DOTs across the US wherever gravel-bed rivers are found.

RESEARCH OBJECTIVES

Phase 1 Objectives

Beta version of SCST software. This proposal funds the first of a two-phased project. Phase 1 includes the development of a +1-dimensional software sediment transport beta model that offers a suite of site calibration options, and state-of-science equations that have been successfully applied to gravel-bed rivers. The equations proposed for inclusion are the multi-fraction approach of Bakke et al. (1999) and Bakke et al. (2017) and the multi-fraction approach of Wilcock and Crowe (2003). We anticipate the software providing the following capabilities at a minimum:

- Incorporate a reach with multiple cross sections;

- Offer several ways to specify and calibrate reach hydraulic relationships;

- Allow simulation of long-term sediment flux using hydrologic (flow-duration) statistics;

- Include the option of allowing the streambed cross sections to evolve according to altered hydrologic statistics and sediment input;

- Visualize model error in order to better assess confidence in model calibration. Error measurement would be compared against optimization parameters in a 2-dimensional plot for fine-tuning or calibrating the parameters used in the bedload equations.

Bakke et al. (1999) and Bakke et al. (2017) incorporate a multi-fraction modeling approach, which offers greater inference power for determining calibration data quality and screening of sample reliability. Bakke et al. (2017) presents a new algorithm for site calibration of the Parker-Klingeman (1982) transport model that is well suited to streams with high sediment loads composed of predominantly gravel, or cases where extrapolation of transport function estimates is necessary. It can be calibrated to either surface or subsurface streambed size distribution, allowing for flexibility in modeling objectives and site stability characteristics.  The multi-fraction model allows inferences about the appropriateness of the sediment equilibrium assumption at a site, which is not possible with a total-load model.  Moreover, this approach results in calibrated values of threshold of motion by grain size rather than for median (D50) size alone.

Wilcock and Crowe (2003) presents a surface-based sediment transport model for mixed sand and gravel sediments; fractional transport rates are referenced to the size distribution of the bed surface rather than subsurface, making the model capable of predicting transient conditions. Critical shear stresses are calculated for each sediment size fraction within the model rather than being specified by the user or set to a single value. Through this method, the model accounts for the hiding function and the effect of sand content on gravel transport rate not included in previous models. The model uses the surface sediment distribution rather than the subsurface to capture the immediate interface between bed surface and sediment in transport.

Phase 2 Objectives

Case studies. Phase 2 efforts include the use of beta SCST software for developing case studies in a range of settings and field data availability, which would also provide critical feedback for refining the software. Case studies would be included in the guidance manual and training materials to illustrate the application of the site-calibration methodology and software in a range of contexts and conditions.

Guidance manual and training materials. Development of practical guidance and training materials would:

- Build on previous efforts (e.g., Wilcock et al, 2009), address critical information gaps, and provide detailed discussion of available models, considerations for model selection, and their appropriate use in real-world circumstances.

- Provide a decision matrix for assessing the need for sediment transport modeling and developing targeted, key questions that aid in identifying the appropriate level of thoroughness and accuracy.

- Inform when and how to conduct sediment transport studies for gravel-bed rivers;

- Provide instruction on using the proposed software model.

- Discuss common sources of error, how to minimize error, and offer modern methods for visualizing errors and inferences about sediment dynamics and data reliability from the calibration process.

- Include case studies for illustration.

Version 1.0 of SCST software. We expect the beta software to undergo refinement following the development of case studies and incorporation of feedback from project partners. The beta SCST software would be published as Version 1.0 as a part of Phase 2 efforts.

Note: If time and funds allow, we would like to explore the viability of integrating of the DREAM models by Cui et al. (2006 a & b) in the SCST software for modeling sediment pulses, which are notoriously difficult to model with available sediment transport equations. Cui et al. (2006 a & b) DREAM models were developed to describe sediment flux (erosion, transport, and deposition) of a large sediment pulse composed of sand and/or sand with a gravel layer on gravel-bed rivers following dam removal. The equations of Cui et al. (2006 a & b) can potentially be applied toward the evaluation of channel hazards and channel response associated with (1) dam-break floods and dam failures, (2) sediment inputs from natural processes such as landslides, wildfires, and volcanic eruptions, and (3) geomorphic responses to climate change, including the transport of sediment pulses associated with glacier retreat (Evans and Clague, 1994; Shakesby and Doerr, 2006; Hoffman and Gabet, 2007; Sklar et al., 2009; James, 2013; Merritts et al., 2013; Pierson and Major, 2014).

WORK TASKS

The project team consists of co-technical leads Cygnia Rapp (Washington State Department of Transportation (WSDOT)) and Paul Bakke (US Fish & Wildlife Service (USFWS)), and a scientific code specialist, to be determined. Ms. Rapp and Mr. Bakke will secure three to four reviewers to the project. The current list of candidates includes Drs. Catalina Segura (Oregon State University), Joanna Crowe Curran (of Wilcock and Crowe (2003)), John Pitlick (University of Colorado, Boulder), Tom Lisle (retired), Wen Wang (San Jose State University), Connor Shea (USFWS, Arcata), Leonard Sklar (University of San Francisco), and Colin Thorne (retired). All candidates are highly regarded in the field of sediment transport modeling.

Phase 1: Develop Beta Version of SCST Software ($99,995)

Task 1:  Conduct 1-day Workshop Retreat. The purpose of the workshop retreat is to maximize efforts in the development of the software package and guidance materials by: (1) defining upfront the desirable software deliverables and functionality to inform the development of the code framework, (2) outlining critical content to ensure the practicality of guidance documentation, and (3) defining content for training materials that coordinates with guidance manual development. The focus group would include the project team, technical reviewers, and technical representatives from state and/or federal partners.

Task 2:  Develop software framework and scientific code. Priorities defined in the Workshop Retreat will inform the development of the SCST software model.

Phase 2: Develop Final Draft Guidance Manual & Training Materials, Publish Version 1.0 of SCST Software ($45,000)

Task 1:  Develop case studies. The project team will test the SCST software via development of case studies. Technical reviewers and project partners will be invited to use the software on their projects. Case studies will provide real world examples of applying the proposed methodology and software in a range of settings and field data availability.

Task 2:  Draft guidance manual and training materials. Outcomes of the Workshop Retreat will refine key content in the user manual and training materials. The project team will review the state-of-science in sediment transport models (published equations and software models), and innovation in field data collection techniques for calibrating sediment transport models.

Task 3:  Solicit feedback from project partners. The project team will solicit feedback from state and federal partners on the software program, user manual, and training materials.

Task 4:  Revise and publish Version 1.0 of software; finalize and publish the guidance manual. The lead agency, WSDOT, will work with participating DOTs to publish Version 1.0 of the software and user manual on state DOT websites and potentially with FHWA.

FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research (for Phase 1) will take 12 months to complete and will require $99,995 from NCHRP. Phase 2 would be funded from other sources, such as state and/or federal partners.

CONTACT PERSON

Jeff Carpenter, P.E.

State Design Engineer

Washington State Department of Transportation

310 Maple Park Avenue SE

Olympia, WA 98501

360-705-7231

360-704-6381 (fax)

carpenj@wsdot.wa.gov

Cygnia Rapp

Environmental and Engineering Programs

Washington State Department of Transportation

310 Maple Park Avenue SE

PO Box 47331

Olympia, WA 98504-7331

360-705-7415

360-705-6833 (fax)

rappcyg@wsdot.wa.gov

REFERENCES

Bakke, P. D., P. O. Basdekas, D. R. Dawdy and P. C. Klingeman, 1999.  Calibrated Parker-Klingeman model for gravel transport.  Journal of Hydraulic Engineering 125, no. 6: 657-660.

Bakke, P.D., L.S. Sklar, D.R. Dawdy and W.C. Wang, 2017. Design of a site-calibrated Parker-Klingeman gravel transport model.  Water, 9(6), 441; doi:10.3390/w9060441

Cui, Y., G. Parker, C. Braudrick, W.E. Dietrich, and B. Cluer. 2006a. Dam removal express assessment models (DREAM). Part 1: model development and validation. Journal of Hydraulic Research 44, no. 3: 291–307.

Cui, Y., C. Braudrick, W.E. Dietrich, B. Cluer, and G. Parker. 2006b. Dam removal express assessment models (DREAM). Part 2: sample runs/sensitivity tests. Journal of Hydraulic Research 44, no. 3: 308–323.

Evans, S.G., and J.J. Clague. 1994. Recent climatic change and catastrophic geomorphic processes in mountain environments. Geomorphology 10: 107-128.

Hoffman, D.E., and E.J. Gabet. 2007. Effects of sediment pulses on channel morphology in a gravel‐bed river. Geological Society of America Bulletin 119: 116‐125.

James, L.A. 2013. Legacy sediment: Definitions and processes of episodically produced anthropogenic sediment. Anthropocene 2 (2013): 16‐26.

Merritts, D., et al. 2013. The rise and fall of Mid‐Atlantic streams: millpond sedimentation, milldam breaching, channel incision, and streambank erosion. Vol. 21, in The Challenges of Dam Removal and River Restoration, edited by J.V. De Graff, & J.E. Evans, 183‐203. Geological Society of America Reviews in Engineering Geology.

Parker, G., and P. C. Klingeman. 1982. On Why gravel bed streams are paved. Water Resources Research 18, no. 5:1409-1423.

Pierson, T.C., and J.J. Major. 2014. Hydrogeomorphic effects of explosive volcanic eruptions on drainage basins. Annual Review of Earth and Planetary Science 42: 469‐507.

Pitlick, J. Y. Cui and P. Wilcock, 2009.  Manual for Computing Bed Load Transport Using BAGS (Bedload Assessment for Gravel-bed Streams) Software.  Gen. Tech. Rep. RMRS-GTR-223. USDA-Forest Service, Rocky Mountain research Station, Fort Collins, CO. 45 pp.

Shakesby, R.A., and S.H. Doerr. 2006. Wildfire as a hydrological and geomorphological agent." Earth‐Science Reviews 74: 269‐307.

Sklar, L.S., et al. 2009. Translation and dispersion of sediment pulses in flume experiments simulating gravel augmentation below dams. Water Resources Research 45, no. 8.

Wilcock, P.R. and J. C. Crowe. 2003. Surface-based transport model for mixed-size sediment.  Journal of Hydraulic Engineering 129: 120-128.

Wilcock, P. R., J. Pitlick and Y. Cui. 2009.  Sediment transport primer: estimating bed-material transport in gravel-bed rivers.  Gen. Tech. Rep. RMRS-GTR-226. USDA-Forest Service, Rocky Mountain research Station, Fort Collins, CO. 78 pp.