Synthesis Needs Statement

Title:  Longer Pavement Life from Increased In-Place Density of Asphalt Pavements

Tentative Scope

In the mid 1990’s, the Strategic Highway Research Program’s development of the Superpave System was completed and adopted by many state highway agencies throughout the country.  The Superpave system was the culmination of a major research effort geared towards identifying and measuring key asphalt binder and mixture properties that can lead to longer lasting asphalt pavements.  In addition to identifying, measuring and specifying critical material properties related to good performance, the Superpave system also led to a general standardization of key binder and mixture testing and specification requirements, which has proven to improve asphalt pavement performance.

While mixture and binder changes introduced with the Superpave system have played a key role with respect to improved pavement performance, it has long been recognized that the level of compaction (in-place density) of an asphalt pavement also plays a major role with respect to its long-term performance.  But unlike the Superpave system, which is captured in AASHTO Standards M 320 and M 323, there is not a standard on how agencies should measure and set criteria for pavement density. Most agencies have developed their own methodologies and acceptance criteria based on past practices and experiences.  Consequently, widespread variability exists throughout the country on how pavement density is measured and even greater differences on how density is specified.  This is in stark contrast to the standardization of the Superpave system with respect to key mixture and binder properties.

The FHWA Asphalt Mixture ETG recently identified pavement density as a critical issue that needs better guidance and direction on a national level. In a limited review of the available literature, many differences were noted nationally with respect to methods of measuring density, test locations in a paving lane, frequency of tests, specified density levels, as well as how the measurement data is applied against the specified minimum acceptance criteria (PWL, averages, lowest single measurement, etc.). A synthesis document summarizing the current state of knowledge and practices regarding in-place density of asphalt pavements would be highly beneficial as a basis for (1) agency decisions concerning in-place density methods and requirements; and (2) identification of the specific knowledge gaps that need to be addressed to provide a complete basis for establishing more comprehensive density standards.

Objectives

The objective of this project is to produce a synthesis summarizing the current state of knowledge of in-place density of asphalt pavements as well as the current practices of agencies regarding how in-place density is measured and specified.  A review of the standard literature (TRB, NCHRP, AAPT, etc.) as well as a survey of state highway agencies will be required. At a minimum, detailed information should be collected on the following:

- Previous or on-going studies on in-place pavement density and longitudinal joint density, including optimal target density levels, the relationship of density to performance, as well as the impact of lift thicknesses on density.

- Typical methods used to measure pavement density (nuclear gauge, non-nuclear gauge, roadway cores), sample sizes; and testing methods used.

- Emerging technologies used to measure pavement consistency during construction.  These methods include thermal imaging, as well as Intelligent Compaction (IC).

- Specification types and requirements, such as: methods of specifying density and if/how they vary by roadway classification; responsibilities for measurement (control vs. acceptance); methods of density measurement; target density levels (percent of control strip, percent of Gmm, percent of lab density, etc.); frequency of testing; as well as acceptance and payment parameters.

This synthesis will serve as the first step in better understanding the current state of practice for pavement density.  With the adoption of Superpave in the 1990s and 2000s pavement rutting has become a much less common problem.  Today durability, predominantly cracking and raveling, has become the focus of concern and research.  While the use of modified binders is known to improve cracking resistance, it also significantly increases the cost of asphalt pavements and may only be affordable for the higher classes of roadways.  The most cost-effective solution to improving durability and performance may be to improve in-place density.

Pavement density is known to improve the mechanical properties of asphalt pavements (e.g. stiffness, tensile strength) and to reduce the rate of aging, thus making the pavement less susceptible to cracking.  If increased compaction (higher pavement density) is achievable, it offers a low-cost method to improve pavement performance.

The key to increasing pavement density is understanding the current state of practice for density of different classes of pavements.  At the same time this synthesis needs to draw together information regarding the role of density and performance on pavement life.

Urgency, Payoff Potential, and Implementation

The life of an asphalt pavement varies widely.  Generally, it can be said that high type applications, such as main highways, have a shorter life than low volume rural or urban roads.  Low volume pavements generally can tolerate higher levels of distress before rehabilitation.  Assuming a pavement life for main highways is 14 years and for low volume roads is 25 years, an average pavement life of 17 years can be assumed.

According to the National Asphalt Pavement Association Annual Asphalt Pavement Industry Survey (IS 138), the 2013 production of asphalt pavement (hot mix and warm mix) was 350 million tons. Using an assumed average price of $70 per ton, this represents a value of $24.5 billion.  If pavement life can be increased by one year (to 18 years), nearly a 6% increase in life occurs.  This represents a value of $1.4 billion.  Even if only half of this value can be captured, it represents a cost “savings” of $700 million per year.

This synthesis is the first step to understand if an opportunity for increased life from pavement density is available.

Estimated Cost and Duration: $75,000 and 12 months

Submitted by:

FHWA Asphalt Mixture ETG

AASHTO SOM 2d