NCHRP 20-7
Proposed Research Needs Statement

AASHTO Committee T-18
Matt Farrar
April 25, 2017

TITLE

CALIBRATION OF LOAD FACTORS FOR THE FAST ACT EMERGENCY VEHICLES EV-2 AND EV-3 FOR LOAD RATING

BACKGROUND / NEEDS STATEMENT

On December 4, 2015, the President signed into law the Fixing America's Surface Transportation Act (FAST Act) (P.L. 114-94), which includes new truck size and weight provisions that effect bridge load rating and posting requirements. Among those provisions is the exemption of emergency vehicles from meeting the nationwide Interstate truck weight limits set forth in 23 U.S.C. 127(a). The emergency vehicles exempted from these weight limits by the FAST Act can create greater load effects in certain bridges than the previous legal loads. Research needs to be undertaken to establish whether existing load factors, multiple presence factors based on likely traffic situations, and exposure intervals as specified in the AASHTO Manual for Bridge Evaluation are appropriate for the emergency vehicles; further these load factors may need to be developed or calibrated to the reliability analysis in the AASHTO Manual for Bridge Evaluation.

RESEARCH OBJECTIVE

The overall objective of this research is to derive load factors for the emergency vehicles using methods similar to that used in the AASHTO Manual for Bridge Evaluation. The load factors to be developed should be calibrated to the reliability analysis in the AASHTO Manual for Bridge Evaluation with appropriate modifications.

WORK TASKS
Tasks anticipated in this project include the following:

- Review the FAST Act provisions regarding emergency vehicles and other available references such as FHWA’s QUESTIONS AND ANSWERS – Load Rating for the FAST Act’s Emergency Vehicles (March 2017) and other FHWA Technical Guidance.

- Perform a standard calibration process consistent with the AASHTO Manual for Bridge Evaluation to determine load factors  corresponding to current resistance factors  for the FAST Act emergency vehicles. Combinations should yield a safety index close to the target value of  = 2.5.

- Establish multiple presence factors appropriate for the FAST Act emergency vehicles using recent traffic data. Include in the calibration the effect of multiple presence factors, discuss or determine the necessity to include emergency vehicles in combination with lane loads for longer spans, the  utilization of striped lanes or positioning the emergency vehicle for maximum effect, and the appropriate use of dynamic load impact, etc.

- Determine through review of the literature and practice whether reducible loads (water) are included in the gross weight of the emergency vehicles and the effect this has on the calibration.

- Make recommendations regarding modifying load factors (load intensity, impact, lane loads, etc.) for the Load Factor Rating (LFR) method approach for load rating in the Manual for Bridge Evaluation.

URGENCY
It is critical that more accurate data be made available to the nation in order to better understand the safety of bridges subjected to FAST Act emergency vehicles and potential for posting numerous bridges on the nation’s transportation system.

FUNDING REQUESTED AND TIME REQUIRED
Funding:  $100,000.00. Time Requirement: 6 months

CONTACT PERSON

Matt Farrar

Idaho Transportation Department

Jeffrey Pouliotte

Florida Department of Transportation

Bala Sivakumar

HNTB Corp

5 Penn Plaza, 6th Floor

New York, NY 10001

212-594-9717

Fax 212-597-4030

bsivakumar@hntb.com