## NCHRP 20-7

## Proposed Research Needs Statement

## AASHTO Subcommittee on Materials

## Moe Jamshidi, Chair

## March 26, 2015

# TITLE

Using Resistivity Measurements to Develop a Formation Factor Specification

BACKGROUND / NEEDS STATEMENT

Electrical resistivity measurements are becoming a potential tool for assessing early and predicting later-age concrete performance.1-4 The existing “best” test method, the ASTM C1202 (Rapid Chloride Permeability, RCPT) has many well-known shortcomings. Several shortcomings are that the test: is slow and time consuming, is destructive, is prone to errors caused by sample heating, fails to capture idiosyncrasies associated with SCMs and has time constraints in mixtures incorporating SCMs.5-7 Despite these limitations, the concrete community has not yet developed and validated a test method that is fast, reliable and user friendly.  Resistivity measurements have been proposed as one potential solution; however they can unfairly provide advantages to select cement sources.  This can be resolved by using a formation factor approach.  This research need statement focuses on the need to develop a far more reliable concept called the formation factor.  The formation factor (F) provides a better assessment of transport properties for use in predicting long-term concrete durability performance, and is related to ratio of the resistivity () of the bulk concrete and resistivity (o) of the pore solution.2, 8-11

RESEARCH OBJECTIVE

The objective of this project is to develop a table of (simple and measureable) formation factors, based on resistivity, for very low, low, moderate, and high permeability concrete mixtures.  The results will also provide for a proposal to replace current w/c limits in specifications.

# WORK TASKS

Tasks anticipated in this project include the following:

- Task 1: Obtain field samples, mixture designs, and constituent materials from various SHAs for surface resistivity measurements, RCPT testing, and pore solution analysis.

- Task 2: Perform resistivity, RCPT, pore solution measurements on laboratory cast concrete of varying composition and w/cm (SCMs, portland limestone cements, and lightweight aggregates).  The incorporation of lightweight aggregates will involve a correction (to be developed in this project) due to their conductivity.

- Task 3: Evaluate the relationships in literature between cement chemistry and pore solution resistivity.

- Task 4:  Develop a draft specification based on the outcomes of this project to inform the end user of how to run the test, collect data and interpret the results.  The data from Tasks 1-3 will be used to develop a table of formation factors for the specification based on resistivity for very low, low, moderate and high permeability concrete mixtures.

# URGENCY

Electrical resistivity is a very hot topic in terms of service-life prediction.  This is evidenced by a myriad of research projects funded by state DOTs and federal dollars and a burgeoning presence in publications, reports, short communications and tech briefs.1-4, 8-10, 12, 13 The community is calling for a specification based on resistivity several SHA’s and FHWA have begun proposing a table with ranges of resistivity values, similar in aim to the current RCPT.  It has been proposed that simply implementing a specification table based on resistivity is a potential major mistake for the concrete community and this approach would allow this to be overcome.  This research is timely as it would enable this error to be avoided allowing the specification to be based on the formation factor, which is faster, more economic, scientifically valid and superior to the current RCPT test.

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $100,000.

### CONTACT PERSON(S)

Tyson D. Rupnow, Ph.D, P.E.

Senior Concrete Research Engineer

Louisiana Transportation Research Center

4101 Gourrier Avenue

Baton Rouge, LA 70808

Office: 225-767-9148

Fax: 225-767-9108

Email: Tyson.Rupnow@la.gov

Mark E. Felag, P.E.

Managing Engineer

Rhode Island Department of Transportation

Two Capitol Hill – Room 022

Providence, RI  02903

Office: 401-222-2524 x-4130

Fax: 401-222-3489

Email: mark.felag@dot.ri.gov

ENDORSED BY:  SOM Chair:  Moe Jamshidi; FHWA: Gary Crawford / Gina Ahlstrom, TRB Chair of AFN00: Mohammad Khan, SOM TS 3a and TRB AFN 40 Chair: Mark Felag,  SOM TS 3a Research Liaison: Robert Horwhat, SOM TS 3b Chair: Mick Syslo, SOM TS 3c Chair: Steven Krebs, TRB AFN 10 Chair: Paul Tikalsky, AFN 20 Chair: Tara Cavalline/Norbert Delatte (incoming April 2015), and AFN 30 Chair: Tyson Rupnow.

REFERENCES

1.	Spragg, R., Villani, C., Snyder, K., Bentz, D., Bullard, J., and Weiss, J., Factors that influence electrical resistivity measurements in cementitious systems. Transportation Research Record, 2013(2342): p. 90-98.

2.	Spragg, R.P., Castro, J., Nantung, T., Paredes, M., and Weiss, W.J., Variability Analysis of the Bulk Resistivity Measured Using Concrete Cylinders, Joint Transportation Research Program, Indiana Department of Transportation and Purdue University pp. 13.

3.	Rupnow, T. and Icenogle, P., Surface resistivity measurements evaluated as alternative to rapid chloride permeability test for quality assurance and acceptance. Transportation Research Record, 2012(2290): p. 30-37.

4.	Isgor, O.B., Ideker, J.H., and Trejo, D., Strategies to Increase the Service Life of Existing Bridge Decks, Oregon Department of Transportation.

5.	Julio-Betancourt, G.A. and Hooton, R.D., Study of the Joule effect on rapid chloride permeability values and evaluation of related electrical properties of concretes. Cement and Concrete Research, 2004. 34(6): p. 1007-1015.

6.	Bentz, D.P., Garboczi, E.J., Martys, N.S., Snyder, K.A., Guthrie, W.S., Kyritsis, K., and Neithalath, N. Virtual testing of concrete transport properties, ACI Fall 2009 Convention, November 8, 2009 - November 12, 2009, New Orleans, LA, United states: American Concrete Institute.

7.	Riding, K.A., Poole, J.L., Schindler, A.K., Juenger, M.C.G., and Folliard, K.J., Simplified concrete resistivity and rapid chloride permeability test method. ACI Materials Journal, 2008. 105(4): p. 390-394.

8.	Weiss, J., Relating Transport Properties to Performance in Concrete Pavements, S. Shields-Cook Ed. Federal Highway Admininstration, TPF-5(179), December 2014,  pp. 6.

9.	Presuel-Moreno, F., Suarez, A., and Lio, Y., Characterization of New and Old Concrete Structures Using Surface Resistivity Measurements  Florida Department of Transportation - Research Center, BD546, RPWO #08, August 1, 2010,  pp. 263.

10.	Jenkins, A., Surface Resistivity as an Alternative for Rapid Chloride Permeability Test of Hardened Concrete, Kansas Department of Transportation Bureau of Research, FHWA-KS-14-15, March 2015,  pp. 39.

11.	Session on Resistivity Measurements of Concrete Transportation Research Board Annual Meeting, January 15, 2015, Washington, D.C.

12.	Farnam, Y., Todak, H., Spragg, R., and Weiss, W.J., The Electrical Response of Mortar with Deicing Salt during Freezing and Thawing. Cement and Concrete Composites, 2015.

13.	Sant, G., Rajabipour, F., and Weiss, J., The influence of temperature on electrical conductivity measurements and maturity predictions in cementitious materials during hydration. Indian Concrete Journal, 2008. 82(4): p. 7-16.