## NCHRP 20-7

## Proposed Research Needs Statement

## Subcommittee on Materials

Technical Section 2b - Asphalt

Eileen Sheehy, Chairman

## March 25, 2015

# TITLE

Effect of Elevation on Rolling Thin Film Oven Aging of Asphalt Binders

BACKGROUND / NEEDS STATEMENT

Laboratories at elevations greater than 4000 feet consistently demonstrate lower aging than labs located below 4000 feet.  This results in higher penetration values, lower viscosities, lower residue DSR values, lower creep recovery values, higher Jnr values, etc.  It results in low AMRL proficiency sample scores, poor correlation in cooperative testing such as the WCTG, and problems correlating with other state labs, asphalt suppliers and private testers.  This effect is evident by evaluation of existing data such as AMRL proficiency samples and cooperative samples.  A report by the Modified Asphalt Research Center at the University of Wisconsin-Madison reviewed existing WCTG data. Their conclusions stated that elevation was a factor affecting reproducibility of PG testing and that the current RTFO procedure needed to be refined to take into account elevation of the laboratory.

A Technical Section 2b AASHTO T 240 Task Force examined this issue back in 2010 and recommended a change in the test procedure to address this issue.  This change would have allowing longer aging times for high elevation labs.  The task force proposal was accepted by technical section ballot.  However, Technical Section 2b endorsed additional research to further study the aging issue.  To date, funding for this research has yet to be approved.

RESEARCH OBJECTIVE

Perform additional research to provide a standardized method for adjusting RTFO times based on elevation.  This research could result in a formula and table similar to that for correcting gauge readings for elevation in R 28, Accelerated Aging of Asphalt Binder Using a Pressurized Aging Vessel (PAV).

# WORK TASKS

Collect existing data from AMRL proficiency samples and WCTG samples to corroborate the 4000 foot elevation threshold.

Develop a work plan to determine RTFO aging times needed at high elevations to produce test results comparable to lower elevations.  Obtain various binders of various PG grades and suppliers, unmodified and modified.  Identify suitable AMRL certified labs which cover different altitudes to conduct the testing.  Labs at low elevations will provide baseline results, while labs at higher elevations will test using multiple RTFO aging times to provide comparison data.

Analyze the test data to produce a standardized method for adjusting RTFO aging times based on elevation.

Write a proposed revision to the existing test procedure, AASHTO T 240, to incorporate this new method.

# URGENCY

This problem has been apparent for years, before the introduction of PG asphalts.  The difference was that with viscosity-graded asphalt cements there was only a maximum on the residue viscosity.  Therefore insufficient aging only made it easier to pass the specification.  However, this has changed with the advent of PG asphalts. Now incomplete aging can result in failing the minimum residue DSR and creep recovery specification as well as the maximum Jnr.  Thus, this issue has taken on a new significance, beyond the problem of low ARML scores and correlation issues.  Incomplete aging in the RTFO due to higher elevations can result in incorrectly failing test results, monetary damages, removal of materials, and the possibility of lawsuits.  Even though only a limited number of states and labs are affected, the results can be significant.  It is an issue that has been ignored and needs to be addressed.

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $99,000.

### CONTACT PERSON

Darin Tedford, Chief Materials Engineer

Nevada Department of Transportation

1263 S. South Stewart St.

Carson City, CA 89712

(775) 888-7520

(775) 888-7501 fax

dtedford@dot.state.nv.us