AASHTO STANDING COMMITTEE ON RESEARCH

AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS

I.	PROBLEM NUMBER

To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Repair of Environmentally Deteriorated Precast Prestressed Concrete Bridge Girders

III.	RESEARCH PROBLEM STATEMENT

Concrete bridge girders exposed to adverse marine environment can experience damage due to multiple deterioration mechanisms, including chemical, physical, and biological attack. This deterioration leads to reductions in mechanical properties, serviceability, and aesthetics of the structure.  One of the major causes of deterioration of prestressed concrete members is chloride-induced corrosion of the prestressing steel.  Prestressed concrete girders exposed to saltwater or saltwater spray exhibit excessive corrosion as compared to other applications. The steel and concrete in prestressed concrete are always in high stress state, and corrosion of steel bars results in loss of capacity and deterioration in a faster pace than the conventional reinforced concrete.

The conventional repair methods of damaged prestressed girders due to over-height truck collisions or similar have proven not to be successful in a marine environment.  Repaired prestressed concrete girders in one of Washington State DOT projects in a marine environment have shown signs of deterioration after only a few years in service.  Similar deterioration of prestressed girders in marine environment has been observed in Oregon State bridge projects.  Research is needed to develop repair methodologies that ensure prolonged service lives of precast prestressed concrete girders in aggressive marine environments.

IV.	LITERATURE SEARCH SUMMARY

No records found based on the search criteria from the Research Information Services (TRIS) http://trisonline.bts.gov/sundev/search.cfm  and http://rip.trb.org/search.

V.	RESEARCH OBJECTIVE

The objective of this research was to develop and evaluate repair methodology to prolong service lives of precast prestressed concrete girders in aggressive marine environments. The research shall focus on the various deterioration mechanisms present in coastal exposures.  The research shall investigate the potential for further deterioration and the effect on chloride ingress in repaired sections under prestressing forces.  In particular, the research focused on the following primary objective tasks:

- Review and summarize the current studies of damaged prestressed concrete girders due to saltwater and aggressive marine exposures.

- Review and summarize the current practices of repairing damaged prestressed concrete girders due to saltwater and aggressive marine exposures.

- Determine and quantify environmental conditions in marine environments as well as design details and construction practices, which may limit service life of reinforced concrete elements.

- Develop repair methodologies suitable for aggressive marine environment, and establish properties of current repairs in use as a basis for comparison with proposed repairs.

- Investigate the potential for further deterioration and the effect on chloride ingress in repaired sections under prestressing forces.

- Provide recommendations and draft design standards for repair methodology for precast prestressed concrete girders.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

Recommended Funding:	$100,000

Research Period:		12-18 months

VII.	URGENCY, PAYOFF POTENTIAL, AND IMPLEMENTATION

This research meets the strategic plan for highway bridges and structures, and many crucial industry needs primarily in the major areas of Bridge preservation and long-term performance. This research will develop repair methodology for environmentally damaged prestressed concrete bridge girders. Increasing the durability of prestressed concrete girders is expected to have a large impact on bridge preservation cost and life cycle.

VIII.	PERSON(S) DEVELOPING THE PROBLEM

DeWayne Wilson, P.E.

State Bridge Asset Management Engineer

Washington State Department of Transportation

P.O. Box 47340

Olympia, WA  98504-7300

Phone: (360) 705-7214

E-mail: WilsonD@wsdot.wa.gov

Contact Person:

Bijan Khaleghi PhD, P.E., S.E. (Member of T-8, T-10, T-11, and T-20)

State Bridge Design Engineer

Bridge and Structures Office

Washington State DOT, P.O. Box 47340

Olympia, WA 98504-7340

Phone: (360) 705-7181

Email: khalegb@wsdot.wa.gov

IX.	PROBLEM MONITOR

To be assigned by AASHTO to monitor the research, if programmed, from inception to completion.  The monitor's final responsibility will entail recommendations to the Standing Committee on Research as to how the research results could be implemented.

X.	DATE AND SUBMITTED BY

August 2, 2014, submitted to AASHTO Standing Committee on Research by:

| Bruce V. Johnson P.E. (SCOBS Vice Chair) State Bridge Engineer Oregon Department of Transportation 4040 Fairview Industrial Drive, SE Salem, OR 97302-1142 Phone: (503) 986-3864 E-mail: bruce.v.johnson@odot.state.or.us | Tom Baker, P.E. (Primary Member) State Bridge and Structures Engineer Washington State Department of Transportation P.O. Box 47340 Olympia, WA 98504-7300 Phone: (360) 705-7207 E-mail: BakerT@wsdot.wa.gov |
| --- | --- |
| Zhengzheng "Jenny" Fu, P.E. Assistant Bridge Design Administrator Louisiana Department of Transportation and Development Phone: 225-379-1321 (office) Phone: 225-938-4669 (cell) E-mail: zhengzheng.fu@la.gov | Sam Fallaha, P.E. Assistant State Structures Design Engineer FDOT- Structures Research Center 2007 E. Paul Dirac Drive Tallahassee, FL 32310 Phone: (850)921-7111 E-mail: Sam.Fallaha@dot.state.fl.us |