Submitted By:  AASHTO SUBCOMMITTEE ON BRIDGES AND STRUCTURES

NCHRP Problem Statement Outline

I.	PROBLEM NUMBER

To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Wind Turbulence-Structure Interaction and Aeroelastic Instability for Long-Span Flexible Girder Systems

III.	RESEARCH PROBLEM STATEMENT

Aerodynamics of long-span flexible girder systems and the contributing effect of the applied wind load is not widely understood nor adequately considered in governing standards.  The AASHTO LRFD Bridge Design Specifications (LRFD) provides rather arbitrary criteria for identifying wind-sensitive structures and no guidance for calculating loading effects due to wind turbulence-structure interaction.

The American Society of Civil Engineering (ASCE) Minimum Design Loads for Buildings and Other Structures/ASCE 7 includes a gust effect factor (G) which accounts for the loading effects in the along-wind direction due to wind turbulence-structure interaction.  For flexible structures, defined in ASCE as structures with a fundamental natural frequency less than 1 Hz, the gust effect factor is calculated based on properties of the structure.  A comparable factor is lacking from the AASHTO Specifications and thus the effects of along-wind turbulence-structure interaction are ignored.

Guidance is also insufficient in regards to aeroelastic instability.  LRFD section 3.8.3.1 specifies that bridges with a width to depth ratio exceeding 30.0 shall be deemed wind-sensitive.  However, the commentary for that section notes that value is "somewhat arbitrary."  Further, that limit is intended for finished structures and is likely not valid for bridges under construction.  Long span flexible bridges, such as steel plate girder bridges, are potentially susceptible to dynamic response and instability during beam erection and prior to achieving composite action with the deck.  There is no guidance for identifying bridges which may be wind sensitive during construction and there are no guidelines for calculating appropriate loading values for these flexible structures.

Experimental test-based data and research are needed to investigate the relationship between system stiffness and applied wind and to improve AASHTO design criteria.

TASKS

Task 1. Review relevant literature, specifications and manuals, ongoing research, current practices and other information canvassing all engineering disciplines to determine the current state-of-knowledge. This information shall be assembled from published and unpublished reports, bridge owners, and others.

Task 2. Summarize the various developments since the publication of the original wind load provisions and identify gaps in the current provisions.

Task 3. Prepare a detailed outline identifying the proposed areas of the AASHTO LRFD Bridge Design Specifications wind load provisions that will require modification or addition. Propose examples to illustrate the proposed revisions.

Task 4. According to the approved outline, develop (1) specification language, commentary and associated graphics for inclusion within future editions of the AASHTO LRFD Bridge Design Specifications; and (2) examples.

Task 5. Make presentations to the AASHTO HSCOBS Technical Committee T-5 Loads and Load Distribution. Revise the proposed specifications and commentary in accordance with project panel and T-5 feedback.

Task 6. Submit a final report describing the entire research effort.

IV. 	LITERATURE SEARCH SUMMARY

N/A

V.	RESEARCH OBJECTIVE

The objective of this research is to propose revisions to the wind load and aeroelastic instability provisions in Section 3—Loads and Load Factors and other related sections of the AASHTO LRFD Bridge Design Specifications.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

$100,000

VII.	URGENCY, PAYOFF POTENTIAL, AND IMPLEMENTATION

The current AASHTO LRFD wind provisions do not adequately address potential aeroelastic instabilities for long-span steel girder systems during construction.

VIII.	PERSON(S) DEVELOPING THE PROBLEM

Dennis Golabek, P.E.

Structures Design Office

Florida Department of Transportation

605 Suwannee Street MS-33

Tallahassee, FL32399-0450

(850)414-4296, Fax:  (850) 414-4955

Christina Freeman, P.E.

Structures Design Office

Florida Department of Transportation

605 Suwannee St., Tallahassee, FL 32399-0450

Phone: (850) 414-4190, Fax: (850) 414-4955

IX.	PROBLEM MONITOR

Susan Hida

California Department of Transportation, Structure Policy & Innovation
Assistant State Bridge Engineer
1801 30th St., MS 9-3/1H
Sacramento, CA 95816
Tel. 916-227-8738
Fax. 916-227-9576
Cell 916-799-9751

Susan_hida@dot.ca.gov

X.	DATE AND SUBMITTED BY