NCHRP 20-07

Proposed Research Problem Statement

TITLE

Developing a procedure for estimating the required sample size for the Highway Safety Manual safety performance function calibration factors

PROBLEM STATEMENT

The predictive procedures included in the Highway Safety Manual (HSM) are known as safety performance functions (SPFs) and are based on observed crashes for a select number of locations in the United States.  For a jurisdiction to be able to fully benefit from applying the HSM SPFs to their region, it is necessary to calibrate these models for local conditions. This model calibration will allow the SPFs to adequately represent unique characteristics of crashes for their region (e.g., weather trends, animal crashes, varying crash reporting thresholds, etc.) and consequently help to refine predicted safety performance resulting in improved safety investment decisions for a transportation agency.

Data collection can be costly and agencies have limited safety funds that need to be used in an efficient and targeted manner, so it is critical that data collection for the calibration effort include cost effective strategies to accomplish this effort. Currently, the HSM recommends a “one-size-fits” all sample size for calibration procedures that requires crash data from 30 to 50 locations with a minimum number of approximately 100 crashes per year.  This information should be evaluated for a study period that extends across multiple years. This recommended sample size is not fully supported by documented research and several agencies that have initiated SPF calibration efforts indicate that this sample size is impossible to obtain due to infrequent crash conditions associated with certain facility types. As an example, during the Oregon calibration effort, the HSM target sample size of 100 crashes per year for multilane rural three-leg stop controlled intersections could not be achieved. These intersection types have infrequent crashes and so the 100 crashes per year threshold is not practical for these unique facilities.  In fact, the entire Oregon database includes only 107 of these intersection types, and collectively the total crash history at these locations is less than this minimum per year threshold.  For many of these under-represented facilities, it may be reasonable to modify the sample size expectations by basing an assumption on average crash history for similar facilities of by using an applicable statistical sample size method unique for the individual facility characteristics.

The soon-to-be-released HSM freeway and interchange chapter uses a discrete choice model to estimate the proportions of individual crash severities rather than a series of SPFs as included in the current HSM chapters. As a result, there is a need to also expand an enhanced calibration procedure and sample size estimation technique to these severity-level models.

Finally, it is feasible that a jurisdiction with a large transportation network, such as a state department of transportation, may need multiple calibration factors for the same SPFs due to extreme regional differences in weather, topography, traffic pattern trends, etc. More guidance is needed so that a transportation agency can identify, where appropriate, this potential need for multiple calibration factors.  Included in this guidance should also be empirically based recommendations about how often an agency should update calibration factors so as to ensure that these factors accurately capture current safety trends.

OBJECTIVES

The objectives of the proposed research are to (1) enhance the HSM sample size and minimum crash thresholds, (2) develop a procedure for the calibration of severity-level models developed using a discrete choice model, (3) identify when it is necessary to develop multiple regional calibration factors for the same SPF, and (4) determine how frequently an agency should update their HSM calibration factors.

RESEARCH TASKS

The above objectives can be accomplished through the following tasks:

- Review the current calibration procedures in the existing and upcoming chapters of the HSM.

- Conduct a search of reports and papers that have used the calibration procedure in the HSM to better understand the problems related to the procedure.

- Based on the findings from Tasks 1) and 2), incorporate required enhancements to the current calibration procedures in the HSM.

- Conduct research on sample size requirements by the facility type. Develop guidelines on sample size required by each facility.

- Develop a procedure for the calibration of severity-level models.

- Determine regional triggers to use as a basis for multiple calibration factors for the same SPF.

- Provide additional guidance about how often an agency should update their calibration factors.

FUNDING AND TIME DURATION REQUESTED

It is estimated that this research will take 12 months and will cost $85,000.

PERSON(S) DEVELOPING THE PROBLEM STATEMENT

Srinivas Geedipally, Ph.D., P.E.

Karen Dixon, Ph.D., P.E.

Dominique Lord, Ph.D., P.Eng.

E-Mail: srinivas-g@ttimail.tamu.edu

k-dixon@tamu.edu

d-lord@ttimail.tamu.edu

With contributions from Geni Bahar, NAVIGATS Inc.

genibahar@navigats.com

PROBLEM MONITOR

Priscilla Tobias, P.E., , Chair AASHTO Subcommittee on Safety Management-Task Group Technical Safety Publication Oversight and Outreach, Illinois Department of Transportation (Email: Priscilla.Tobias@Illinois.gov)

John Milton, Ph.D., P.E., Chair TRB Highway Safety Performance Committee, Washington State Department of Transportation (Email: MiltonJ@wsdot.gov)

DATE AND SUBMITTED BY

April 1, 2013

AASHTO Safety Management Subcommittee,

Task Group for Technical Safety Publication Oversight and Outreach,

TRB Highway Safety Performance Committee