Proposed Research Needs Statement

Date: April 14, 2016

# TITLE

ASSESSMENT OF JOINTED CONCRETE FAULTING COLLECTION AND ANALYSIS

# BACKGROUND/ Needs Statement

In the last few decades, state DOTs and other agencies have measured faulting of transverse concrete joints manually (either measuring joint height differences or with a device such as the Georgia fault meter), or in automated fashion (for example, point lasers or 3D technology).  Many highway agencies have shifted from manual to automated since other pavement management system (PMS) data are also being collected in an automated fashion, such as rutting, cracking, and roughness, it makes sense to collect faulting at the same time.  However, the accuracy of automated means of network-level faulting measurement has been mixed.  Accurate network-level faulting detection method is crucial to a maintenance program. The analysis of faulting data is also crucial to select proper maintenance projects.  If accurate faulting data is not available or proper analyzed, treatments such as grinding might not be triggered at the appropriate time or location in a pavement’s life cycle.  In addition, automated methods, as opposed to manual, keep personnel off the roads, resulting in greater worker safety.

Some methods of automated faulting detection and data processing have been developed in recent years (such as Florida’s and Mississippi’s methods, and TxDOT 3D line laser sensor method), and AASHTO R36 outlines fault measurement concepts.  TPF-5(354) pooled fund study has a current study on longitudinal profiles.  The TPF-5(299) pooled fund study wishes to investigate the accuracy of automated faulting collection methods and look into ways to analyze faulting data, possibly coordinating with TPF-5(354).

# Research OBJECTIVE

The objective of this study is to perform a literature review to identify current methods of faulting data collection and processing and assess their appropriateness and accuracy.  In addition to collection and processing, this study focuses on faulting analysis as well.  Most of the automated faulting methods were developed when point lasers were the standard of practice.  Nowadays 3D technology may provide more accurate method for detecting, locating, and measuring faulting.  Ideally, a detection and processing algorithm would be technology-independent.  This study should also investigate the following:

1) The impact of surface texture to current faulting measurement.

2) The effect of travel speed.

3) The effect of data collection and analysis due to sensor type (3D, line or point).

4) Methods/algorithms to processing and analyzing faulting data.

The deliverable will be a final report detailing the literature search and information found.

# Work TASKS

- Summarize the existing methods of automated faulting data collection and processing, as well as the effects of surface texture, travel speed and sensor type (3D, line or point).

- Review how faulting data is aggregated and used to make decisions regarding project selection.

- Survey of DOTs’ and other agencies’ current faulting data collection and analysis practices

- Final report outlining findings.

# URGENCY

It is crucial to perform this study now as faulting is an important component of MAP-21 requirements.  This study will result in the following anticipated benefits:

- Triggering of faulting-related projects the appropriate time, which enables DOTs to enhance pavement performance and save repair costs.

- Increase in automated faulting collection, keeping agency and vendor personnel off roads, resulting in greater safety.

- Ability to compare faulting data state-to-state.

# ESTIMATED FUNDING REQUIREMENTS

It is estimated that this research will take 12 months to complete and will require $100,000.

References

AASHTO R 36-13—Standard Practice for Evaluating Faulting of Concrete Pavements

AASHTO M 328—Inertial Profiler

AASHTO R 56—Certification of Inertial Profiling Systems

AASHTO R 57—Operation of Inertial Profiling Systems

ASTM E867—Standard Terminology Relating to Vehicle-Pavement Systems

Miller, J., and W. Bellinger, Distress Identification Manual for the Long-Term Pavement

Performance Program (Fourth Revised Edition), FHWA-RD-03-031, U.S. Department of

Transportation, Federal Highway Administration, Springfield, VA, June 2003.

Miller, J., and W. Bellinger, Distress Identification Manual for the Long-Term Pavement

Performance Program (Fifth Revised Edition), FHWA-HRT-13-092, U.S. Department of

Transportation, Federal Highway Administration, McLean, VA, May 2014.

Chang, G., J. Watkins, and R. Orthmeyer. “Practical Implementation of Automated Fault

Measurement Based on Pavement Profiles.” Presented on December 5, 2011 at International

Symposium on Pavement Performance: Trends, Advances, and Challenges. STP 1555. ASTM

International, West Conshohocken, PA, published in 2012.

Chang, G., S. Karamihas, R. Rasmussen, D. Merritt, and M. Swanlund. “Quantifying the Impact

of Jointed Concrete Pavement Curling and Warping on Pavement Uneveness.” Proceedings of

the Sixth Symposium on Pavement Surface Characteristics (SURF), Slovenia, 2008.

Federal Highway Administration, ProVAL software Automated Faulting Module

Mississippi DOT. (James C. Watkins, P.E.) BATCHCALCFAULT Software User Guide 3.00, March 2010.

Nazef, A., A. Mraz, S. Iyer, and B. Choubane. “Semi-Automated Faulting Measurement

Approach for Rigid Pavements Using High-Speed Inertial Profiler Data.” Transportation

Research Board 88th Annual Meeting, Washington, DC, 2009.

Nazef, A., A. Mraz, S. Iyer, and B. Choubane. “Alternative Validation Practice of an Automated

Faulting Measurement Method.” In Transportation Research Board Records: Journal of the

Transportation Research Board, No. 2155, Transportation Research Board, Washington, DC,

2010, pp. 99–104.

Mraz, A., A. Nazef, H. Lee, C. Holzschuher, and B. Choubane. “Precision of FDOT Automated

and Manual Faulting Measurement Methods.” Transportation Research Board, Paper 12-1376,

91st Transportation Research Board Meeting, Washington, DC, 2012.