AASHTO STANDING COMMITTEE ON RESEARCH

AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS

NCHRP Problem Statement

I.	PROBLEM NUMBER

To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Anchor Rod and End Plate Bolted Connection Tightening Procedures for Highway Sign, Signal, and High Mast Light Structures

III.	RESEARCH PROBLEM STATEMENT

Field inspections of base plate anchor rods in highway sign, signal and high mast lighting structures (ancillary highway structures) have identified loose and missing anchor rod nuts as a recurring installation defect.  Similarly, bolted arm-to-pole end plate connections are often found with loose bolts, or even broken or missing bolts.  These improperly installed connections produce increased stresses on adjacent anchor rods or bolts as well as having reduced fatigue resistance.  The resulting redistribution of connection loads also increases stresses in the pole to base plate, or arm to end plate welds that can lead to fatigue cracking.  Loose connections have been a contributing factor in ancillary structure failures.

Unlike erection of bridges, these ancillary highway structures are often erected/installed by contractors with limited knowledge of proper anchor rod or structural bolt installation procedures.  In addition, tightening of large diameter anchor rods may require specialized hydraulic wrenches.  While the FHWA manual “Guidelines for the Installation, Maintenance and Repair of Structural Supports for Highway Sign, Luminaires, and Traffic Signals” provides installation guidance, this may not contain sufficient detail for those with limited structure erection experience.

The problem in practice is how the erector/installer achieves the required preload.  For anchor rods, this is complicated by the size of rods involved, and hence required high tightening load.  When end plates are attached by through bolting, standard tightening procedures, i.e. turn-of-nut, may be used.  However, many of these connections use tapped holes in the pole connection plate.  Since the plate has different properties than a hardened nut, tightening procedures may need modification.

The proposed project would utilize the existing body of knowledge on connection performance to prescribe installation procedures to assure the desired performance is achieved.  Should gaps in existing knowledge be identified, recommendations for specific research needs could be developed.

IV.	LITERATURE SEARCH SUMMARY

The relation between anchor rod fatigue resistance and proper tightening to develop rod pretension has been investigated by Frank(1) and Dusel et al.(2).  The beneficial effect of pretension was clearly established.  Similarly, the beneficial effect of pretension in high strength bolts, as commonly used in end plate arm to post connections is well established.

Other studies (Keating et al., Abraham and Hodge, Richards) concur in the importance of achieving proper preload and typically recommend turn-of-nut approach.  However, detailed procedures, and potential limitations, of achieving this nut rotation in the field are only addressed in a limited fashion.

V.	RESEARCH OBJECTIVE

This project is intended to produce a “pocket” type installation manual for tightening anchor rods and end-plate connection bolts.  The installation manual would contain detailed step-by-step installation/tightening procedures intended for use by field construction personnel and involve specific guidance on use of any specialized tools.  Proper tightening of new installation as well as retightening of loose anchor connections would be provided.  It would also include required pre and post installation quality control measures.  The manual could be given to installers as well as DOT field staff.

The research is expected to involve the following phases and tasks:

Task 1:	Review existing research on anchor rod tightening effect on connection performance 	considering the typical double-nut moment base plate connection used in ancillary	structures.

Task 2:	Review existing literature on tightening of high strength bolts installed in tapped “blind 	holes” in steel plate.  Of particular interest is galvanized A325 bolt behavior.

Task 3:	Depending on the results of Task 2, conduct physical tests on full size samples of 	galvanized A325 bolts installed into drilled and tapped holes in Grade 50 steel plate to 	determine if turn-of-nut rotation can be achieved without thread failure.  Also determine 	rotation at thread failure.  Develop installation guideline based on test results.

Task 4:	Conduct brief survey, maximum 10 questions, of current state DOT requirement on their 	anchor rod and end plate bolt tightening requirements.  Anticipate assistance of AASHTO 	committee T-12 in survey distribution.

Task 5:	Interview contractors on methods they use in tightening similar connections.

Task 6:	Investigate available tools and aids (i.e. DTI’s or similar) to achieve properly tightened 	connections.  Prepare summary of literature reviews and surveys and installation/tightening 	manual.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

Recommended Funding:

Estimated funding for this project is $100,000 plus any direct travel expenses for project review meetings.  This project is proposed to be completed as a Task under NCHRP 20-07.

Research Period:

Estimated research period for this project is 12 months.

VII.	URGENCY, PAYOFF POTENTIAL, AND IMPLEMENTATION

- The occurrence of improperly tightened anchor rod nuts and end plate bolts continues to be a recurring problem for in-service structures and is known to contribute to failures.  The primary follow-on activity would be an awareness program through TRB, AASHTO, and technical presentation to assure the installation manual is utilized by owners.  The manual might also be proposed for inclusion in the AASHTO Specifications for Ancillary Structures.  This information may also be of interest for other types of transportation structures with similar connection details.

VIII.	PERSON(S) DEVELOPING THE PROBLEM

Mr. Michael Garlich, P.E., S.E.

Collins Engineers, Inc.

123 N. Wacker Dr., Suite 900

Chicago, IL 60606

IX.	PROBLEM MONITOR

Dr. Xiaohua “Hannah” Cheng, P.E.

New Jersey Department of Transportation

Past Chair, TRB AFF10(1) Subcommittee on Traffic Structures

X.	DATE AND SUBMITTED BY

August 6, 2015

Mr. Barry A. Benton, P.E.

State Bridge Engineer

Delaware Department of Transportation

Chair, SCOBS T-12 Technical Committee on Structural Supports for Signs, Luminaries and Traffic Signals

References:

(1) Frank, K.H., “Fatigue Strength of Anchor Bolts,” Journal of the Structural Division, ASCE, Vol. 106, No. ST6, 1980.

(2) Dusel, J.P., et al., Determination of Fatigue Characteristics of Hot-Dipped galvanized A307 and A449 Anchor Bars and A325 Cap Screws, State of California, Department of Transportation, Division of Engineering Services, Sacramento, CA 1984.

(3)  Richards, J.H., “Turn-of-the-Nut Tightening of Anchor Bolts”, Thesis submitted as part of ASCE requirements, Texas A&M University, 2004

(4)  James, et al., “Tightening Procedures for Large Diameter Anchor Bolts”, Texas Transportation Institute, Texas A&M University, 1997, FHWA/TX-98/1472-1F