NCHRP 20-7

Proposed Research Needs Statement

TITLE

Adjustments to the Superpave Volumetric Mixture Design Procedure for Selecting Optimum Asphalt Content

BACKGROUND

Most state highway agencies (SHAs) currently use the Superpave (Superior Performing Asphalt Pavements) volumetric mixture design method as documented in AASHTO R 35 “Superpave Volumetric Design for Hot Mix Asphalt (HMA)” and AASHTO M 323 “Superpave Volumetric Mix Design.”  Superpave mixture design was first introduced in 1993 with the completion of the Strategic Highway Research Program (SHRP).  The Superpave system introduced a new compactor, the Superpave gyratory compactor, for densifying mixes in the laboratory.  It also includes requirements for the constituent materials used in the asphalt mixture design to meet specific volumetric requirements.  The volumetric mixture design is used to determine the optimum asphalt content for desired in-place air voids and long-term performance.

There has been some concern by various SHAs that the Superpave mixture design system produces asphalt mixes that are “too dry” (too low of an optimum asphalt binder content) that may be resulting in durability issues.  In many instances requirements in these AASHTO standards have been changed by most SHAs individually based on their experience.  The requirements that were most commonly changed included the design air voids, minimum voids in the mineral aggregate (VMA) and/or the design gyrations.

To provide guidance in making changes to the AASHTO standards, the FHWA Asphalt Mixture Expert Task Group recommended agencies perform an independent evaluation prior to making any adjustments in compaction levels from the AASHTO R 35 standard.  The evaluation would include the effect of the proposed changes in gyration level to performance for typical aggregates, binder, and mixture designs (FHWA Tech Brief 2010).

OBJECTIVE

The work proposed will develop a summary of successful approaches currently being used by SHAs to make adjustments to the Superpave volumetric mixture design to obtain higher asphalt binder contents.  This increase in optimum asphalt content will be demonstrated with a limited laboratory experiment to compare the successful approaches with the current AASHTO standards.  This additional information will then be documented for inclusion as an appendix to AASHTO R 35 and/or AASHTO M 323.

URGENCY

Most SHAs are making many changes to AASHTO M323 and R35 to obtain higher optimum asphalt contents.  Some SHAs have been successful and others have not.  The information needs to be gathered and shared so SHAs can make informed decisions about their modifications.

Examples that are effectively increasing the optimum asphalt include the selection of the optimum asphalt content using the “gyratory regression” or “air void regression” techniques.  Increasing the minimum voids in the mineral aggregate (VMA) is another common modification.  Another effective example of a change is Superpave 5 (Hekmatfar 2013) where asphalt mixtures are designed to be more compactable in the field, so they can be compacted to the same density as the laboratory mixture design.  Further, there are examples of best practices for implementation such as using SHA-only testing of the bulk specific gravity of the aggregate (Gsb) for calculation of the VMA.

Along with these successful changes, there are also SHAs that have made changes that were not successful at increasing the optimum asphalt content.  Simply designing at lower gyration or lower air voids is not effective.  If aggregate gradations were held constant, these methods could work.  However, in a low-bid environment, the contractors are making changes to gradations and dust content in which asphalt mixtures remain at low optimum asphalt contents.

WORK TASKS

This research project will result in the:

- survey of all SHAs to determine the current adjustments and use of the Superpave volumetric mixture design.  It is anticipated that there will be more detailed interviews of several of those that are identified to have more success in making changes that result in higher asphalt binder contents,

- review of literature for the development and state-of-the-practice for adjustments to the Superpave volumetric mixture design,

- develop a summary of successful practices that could be used as an appendix in AASHTO R 35 and/or AASHTO M323,

- devise and conduct a laboratory experiment comparing the Superpave mixture design process with the successful practices identified in the survey to demonstrate the effectiveness of the alternate procedures and quantify the increased asphalt contents, and

- prepare a final report that documents results, summarizes findings, draws conclusions, and presents the (a) proposed appendices and (b) results of the limited laboratory comparison.

ESTIMATED COST AND SCHEDULE

Recommended Funding: $100,000
Research Period: 12 months

CONTACT PERSONS

Tim Aschenbrener, P.E.

FHWA Office of Infrastructure

Senior Asphalt Pavement Engineer

12300 W. Dakota Avenue, Suite 280

Lakewood, CO 80228

Office: (720) 963-3247

timothy.aschenbrener@dot.gov

Ross “Oak” Metcalfe, P.E.

Montana Transportation Department

Materials Bureau

Testing Engineer/Physical Test Section Supervisor

Office: (406) 444-9201

rmetcalfe@mt.gov

REFERENCES

Federal Highway Administration Tech Brief, A Review of Aggregate and Asphalt Mixture Specific Gravity Measurements and Their Impacts on Asphalt Mix Design Properties and Mix Acceptance, Office of Pavement Technology, FHWA-HIF-11-033, December 2010, 39 pages.

Hekmatfar, Ali; McDaniel, Rebecca S.; Shah, Ayesha; and Haddock, John E. (2013). “Optimizing Laboratory Mixture Design as It Relates to Field Compaction in order to Improve Hot-Mix Asphalt Durability.” Joint Transportation Research Program Technical Report, SPR-3624, Purdue University. 51 pages.