# TITLE

# Corrosion Protection Comparison of W Beam Guardrail using Hot Dip verse continuously Galvanized Guardrail Sections.

BACKGROUND / NEEDS STATEMENT

All State highway agencies use galvanized guardrail as part of roadway safety elements.  A number of State Departments of Transportations have allowed W-beam guardrail elements to be continuously galvanized as well as hot dipped galvanized. W-beam guardrail elements utilizing the hot dip batch galvanization process are first fabricated (shaped, cut, holes punched,) and then hot dip galvanized. In contrast, continuous galvanization takes place before the guard-rail struts are fabricated with rolled steel being continuously galvanized followed by the fabrication process (cut, holes punched, shaped) which results in bare steel in the vicinity of the cut edges at the end and punched holes that are exposed to weathering.

There is no existing research that directly compares the corrosion resistance and life cycle of these two galvanization processes.  The potential for either galvanization process to have a reduced life cycle potentially means highway agencies would have to spend additional money to replace guardrail sections that could have lasted longer if one of the galvanization processes proves to have a longer life cycle. The accelerated testing will speed up the data collection comparing the two galvanization processes versus waiting for field test comparisons which could take a number of decades to develop information.

Literature Search Summary

A search found no existing research papers in the TRIS and Research in Progress databases yielded no results for accelerated testing on corrosion of guardrail utilizing the continuous and hot dipped process. The BUNDESANSTALT FUER STRASSENWESEN in Germany is performing a research project on Continuously Galvanized Guard-Rail Struts, but this research appears to be field based research and is not accelerated testing. The proposed accelerated testing will speed up the data collection comparing the two galvanization processes versus waiting for field test comparisons which could take a number of decades to develop information.

RESEARCH OBJECTIVE

The objective of this research is to compare the continuous galvanization process corrosion resistance to the hot dip galvanization process utilizing accelerated corrosion testing.

# WORK TASKS

Tasks anticipated in this project include the following:

- Identify what accelerated corrosion testing are going to be done to compare corrosion rates utilizing industry standards such as ASTM, AASHTO or GM accelerated corrosion tests.

- Identify time frame to run each of the accelerated test.

- Identify sample size, galvanization methods, and procure samples to test in accelerated test chambers.

- Identify protocol to track corrosion rates with measurements, charts, visual comparisons, etc.

- Run accelerated corrosion testing on both hot dip and continuously galvanized samples tracking corrosion and thickness of galvanization coatings over time.

- Produce report comparing corrosion rates utilizing both continuous galvanization and hot dipped galvanization from multiple tests.

# URGENCY

The accelerated testing will speed up the data collection comparing the two galvanization processes versus waiting for field test comparisons which could take a number of decades to develop information.

### FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 9 to 12 months to complete and will require $90,000.

### CONTACT PERSON

Tom Baker, P.E.

PO Box 47365, Olympia WA, 98504-7365

Phone: (360) 709-5401

Fax: (360) 709-5588

Email: bakert@wsdot.wa.gov

Kurt Williams, P.E.

PO Box 47365, Olympia WA, 98504-7365

Phone: (360) 709-5410

Fax: (360) 709-5588

Email: willikr@wsdot.wa.gov