Fall 20-07 Proposal

Submitted by the Subcommittee on Bridges and Structures

Title: 
Posting Practices for Specialized Hauling Vehicles (SHVs)

Submitted by: Matthew M. Farrar Chair AASHTO Technical Committee T18 Idaho Department of Transportation P.O. Box 7129, Boise, ID 83707 E-mail: matt.farrar@itd.idaho.gov Telephone number:208-334-8256

Scope:

MBE 6A.8.2 notes that when the maximum legal load under State law exceeds the safe load capacity of a bridge, restrictive load posting shall be required. The live load to be used for posting considerations should be any of the typical AASHTO legal loads given below or state legal loads:

- Type 3, Type 3S2, Type 3-3, or lane loads, for routine single and combination commercial vehicles, and

- A single Type SU4, Type SU5, Type SU6, or Type SU7 for single-unit specialized hauling vehicles.

The three AASHTO Trucks Type 3, Type 3S2 and Type 3-3 have been in use for posting since the 1970s and are the most common trucks used on posting signs with truck symbols. The four single-unit posting trucks SU4, SU5, SU6, and SU7 were adopted as posting loads by AASHTO in 2005 to model the short wheelbase multi-axle Specialized Hauling Vehicles (SHVs) that are becoming increasingly more common in many states. It is reported that the use of these SHVs as posting vehicles on signs has not been widely implemented by the states, though many states have included these SHV trucks in their library of rating vehicles. As the number of posting vehicles increase it becomes increasingly difficult to show all the truck symbols on a single posting sign. The use of the SU4 tri-axle dump truck is perhaps the most common SHV used for posting purposes in some states. This points to the need for more innovative strategies for posting of bridges that will balance the need for posting signs that are simple to read and posting weights that are not unnecessarily restrictive. Additionally, the regulatory signing for posting shall conform to the requirements of the Manual on Uniform Traffic Control Devices (MUTCD) or other governing regulations. The MUTCD allows postings based on truck symbols or based on a single tonnage, which is commonly used for posting local bridges. The extent to which state DOTs and other bridge owners have adapted their posting practices and posting signs to accommodate the SHV trucks needs to be investigated and the best practices should be shared with agencies that are still be in the process of implementing SHV posting procedures for their bridges.

This research fills an immediate need for additional guidance on SHV posting and signing practices. It will provide a review of the state-of-practice with regard to SHV postings. Instead of each state developing their own procedures for SHV postings, it would be best to develop general procedures that can be used by all the states.  This will result in more uniform posting signs and promote greater compliance of postings. Therefore, this NCHRP Synthesis project is proposed to develop model procedures for SHV postings that will serve as a guide for all states to follow.

Work to be performed is to include:

  - Locate and assemble documents on SHV posting procedures procedures from DOTs and other US bridge owners.

  - Review AASHTO, FHWA and state load rating and posting manuals and directives.

  - Conducting a survey of bridge owners and engineers on this topic.

  - Document the posting signs commonly used for SHV load restrictions.

  - Contact owners and engineers to learn about what works and what problems remain largely unsolved with regard to SHV posting. Seek input on how current procedures could be improved.

  - Organize, evaluate, and document the information that is acquired in the study.

  - Prepare a report detailing the findings of the study.

  - Prepare a stand-alone document that summarizes the best practices for SHV postings.

FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $100,000.

CONTACT PERSON 
Matthew M. Farrar

Chair Technical Committee T18

Idaho Department of Transportation

P.O. Box 7129, Boise, ID 83707

E-mail: matt.farrar@itd.idaho.gov

Telephone number: 208-334-8256