NCHRP 20-7

Proposed Research Needs Statement

Subcommittee on Materials, Technical Section 2c

Allen Myers

3/27/15

TITLE

Develop criteria that establish the amount of energy required to maintain fully-animated particles of loose asphalt within the test procedure AASHTO T 209.

BACKGROUND / NEEDS STATEMENT

The intent of the theoretical maximum specific gravity test procedure (also known as Gmm) is to remove all air from a loose asphalt mix sample, thus establishing the theoretical maximum specific gravity. During the test, the sample is placed in a container, covered with water, a vacuum pulled and vibrated to minimize any air remaining in the sample.  If there is any air in the sample at the end of the test, then the test results will reflect something less than the true Gmm.

The challenge is establishing a process that ensures the removal of all air since an individual is unable to visually see through the sample to confirm all air has been removed.  Mechanical shaking methods have already demonstrated a more true and repeatable process over manual shaking methods. Recent research conducted in NCHRP Project 10-87(01), “Precision Statements for AASHTO Standard Methods of Test,” by the AASHTO Asphalt Materials Reference Laboratory (AMRL) concluded the difference in air voids from manual and mechanical agitation was in the range of 0.2% to 0.4%.

However, the research fell short by not determining the optimum mechanically-derived Gmm. The research used single amplitude and frequency settings throughout the test, which does not guarantee obtaining the true Gmm. Using the research’s approach, a sample will become animated, settle (interlocking the particles) and begin rotating.  It becomes more difficult for any additional air to be released once the particle interlock occurs.

RESEARCH OBJECTIVE

Establish a mechanical shaker criterion to determine the optimum amplitude and frequency, defined as achieving full animation of individual sample particles, for each unique mix sample.  Once it achieves the initial setting, additional criteria is required to continuously modify the amplitude and frequency

whenever the material fails to maintain full animation throughout the test of the individual particles within the sample.

This represents the first phase to refining the use of mechanical shakers within AASHTO T 209. Additional phases will be required, such as verifying the criteria has merit on various asphalt binders and aggregates sizes/sources. It will also be necessary to reestablish repeatability and reproducibility Gmm values when using the new criteria.

WORK TASKS

Tasks anticipated in this project include the following:

	Enlist the services of a mechanical engineer or other expertise to ensure emphasis is placed on the determination of energy input by the mechanical devices and includes mechanical energy analysis approach/criteria as one of the deliverables (i.e. how to measure, what equipment can be used to measure, how is it measured and documented).

	Establish a set of criterion to produce continuous full animation of individual particles throughout the test procedure vacuum step.

	Create clear and concise instructions that can be incorporated into the current AASHTO T

209 for mechanical shaker manufacturers to meet the established criterion.

URGENCY

Within Superpave, the Gmm impacts both air voids and density, which influences millions of dollars in price adjustments every year. This research will establish criterion for a mechanical shaker designed to minimize the risk that the test result is less than the actual Gmm.

FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $100,000.

PERSON(S) DEVELOPING THE PROBLEM

Developed in cooperation with AASHTO Subcommittee on Materials, Technical Section 2c on Asphalt- Aggregate Mixtures.

Matthew Corrigan, P.E.	Rick Kreider, P.E.

Federal Highway Administration	Bureau Chief of Research

1200 New Jersey Ave SE	Kansas Department of Transportation

Room E73-465	Materials and Research Center

Washington, DC 20590	2300 Van Buren

Phone: 202.366.1549	Topeka, KS

Email:  matthew.corrigan@dot.gov	Phone: 785-296-1195

Email:  rickk@ksdot.org

CONTACT PERSON

Allen H. Myers, P. E. Director

Division of Materials

Department of Highways

Kentucky Transportation Cabinet

1227 Wilkinson Boulevard

Frankfort, Ky.  40601-1226

Telephone:  502-564-3160

Fax:  502-564-7034

E-mail:   Allen.Myers@ky.gov