- NCHRP 20-07

- Proposed Research Needs Statement

AASHTO Subcommittee on Bridges and Structures

Chair, Gregg Fredrick, Wyoming Department of Transportation

- April 2015

- TITLE

Incorporation of AASHTO Guide Specifications for LRFD Bridge Seismic Design into the AASHTO LRFD Bridge Design Specifications

- BACKGROUND

Currently the AASHTO LRFD Specifications carries two seismic design provisions.  The AASHTO LRFD Bridge Design Specifications address the force-based seismic design while the AASHTO Guide Specification for LRFD Bridge Seismic Design addresses the displacement-based design.  Both seismic design specifications are using a 1000-yr return period earthquake ground motion, with seven percent probability of exceedance in 75 years, and both specifications offer a separate set of bridge seismic design guidelines.

In the AASHTO LRFD Bridge Design Specifications each bridge shall be assigned to one of the four seismic zones 1 through 4.  The AASHTO Guide Specifications for LRFD Bridge Seismic Design Specifications establishes four Seismic Design Categories (SDCs) A through D.  The seismic provisions of the Guide Specifications are displacement-based design procedures, while the seismic provisions of the AASHTO LRFD Bridge Design Specifications are arbitrary force-based response modification factor “R-Factor” method.

The AASHTO Guide Specification is more comprehensive than the AASHTO LRFD Bridge Design Specifications.  The displacement-based approach incorporates a simplified implicit displacement check procedure and a more rigorous pushover assessment of displacement capacity. The guide specifications also include a detailed guidance on earthquake-resisting elements and systems, global design strategies, demand modeling, capacity calculation, and liquefaction effects. The capacity design procedures of the Guide Specifications include prescriptive detailing for plastic hinging regions and design requirements for capacity protection of those elements that should not experience damage.

The integration of AASHTO bridge seismic design specifications into one provides consistency in seismic design.  The displacement-based seismic design allows the designers to better understand the behavior of a bridge to seismic loads and more predictable performance of bridges in seismic events.  The displacement-based seismic design could results in most cases to more economical structures since designers have the ability of incorporating material properties and monitor the displacement of the structure to the desired ductility.  The integration of two seismic specifications removes the design differences between the two specifications in concrete, steel, walls, foundation, etc.

RESEARCH OBJECTIVE

The objective of this research is to integrate the two AASHTO seismic specifications into one unified displacement-based seismic design specifications.  This research will identify and compare the seismic design articles of both AASHTO Specifications, and thoughtfully replace the force-based seismic design provisions AASHTO LRFD Bridge Design Specifications with corresponding displacement-based design seismic provisions of the AASHTO Guide Specifications.  The integration should be comprehensive and address all seismic design provisions related to concrete, steel, foundations, walls, etc. throughout the specifications. The research should also address the ambiguity of seismic zones vs. seismic design categories between the two seismic design specifications.   The specific aims of this research are summarized in the following tasks:

- Identify all seismic design articles of both AASHTO Specifications;

- Compare the seismic design provisions of the LRFD Specifications with seismic design provisions of the specifications.  Develop numerical example for this comparison;

- Replace the seismic design provisions of the LRFD Specifications with seismic design provisions of the specifications;

- Develop comprehensive design example for typical concrete and steel bridges;

- Submit a final report documenting the entire research effort, and recommended revisions to the LRFD Bridge Design Specifications for AASHTO SCOBS consideration.

- URGENCY

Owners, Designers, and the bridge community will have the information they need to make better informed decisions regarding the design, construction, and specifications for seismic design of bridges that are economically feasible and perform as intended.

- FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $75,000.

- CONTACT PERSON

Bijan Khaleghi, Ph.D., P.E., S.E.

State Bridge Design Engineer

Bridge and Structures Office

Washington State DOT, P.O. Box 47340

Olympia, WA 98504-7340

Phone: (360) 705-7181

Email: khalegb@wsdot.wa.gov