- NCHRP 20-7

- Proposed Research Needs Statement

AASHTO Subcommittee on Bridges and Structures

Chair, Gregg Fredrick, Wyoming Department of Transportation

- April 2015

- TITLE

Develop LRFD Design Specifications for Widening of Existing Bridges in Seismic Regions

- BACKGROUND

Widening of existing bridges in seismic regions is often challenging, specifically when it comes to determining how to address elements of the existing structure that do not meet current bridge design standards.   The LRFD bridge design specifications do not provide guidance on how to treat existing structures that are being widened. The IBC is often referenced for design of most structures other than bridges. The requirements of the IBC are often taken as an acceptable level of safety that meets the expectations of the public.

Bridge owners often establish internal policies of “Do No Harm” for bridge widening projects in seismic regions.  The “Do No Harm” policy requires the bridge engineer to compare existing bridge element seismic capacity/demand ratios for the before widening condition to those of the after widening condition.  If the capacity/demand ratio is not decreased, the widening can be designed and constructed without retrofitting existing seismically deficient bridge elements. The decision to retrofit these elements is left to the owner and is based on funding availability.

The “Do No Harm” policy allows bridge widening projects to be completed without addressing existing seismic risks, provided “No Harm” is done to the existing structure. The existing seismic risks are left to be addressed by a bridge seismic retrofit project. The geotechnical designer should evaluate the potential for differential settlement between the existing structure and widening structure. Additional geotechnical measures may be required to limit differential settlements to tolerable levels for both static and seismic conditions. The bridge designers shall evaluate, design, and detail all elements of new and existing portions of the widened structure for the differential settlement.  Design specifications and guidance for widening of existing bridges in seismic regions are needed.

RESEARCH OBJECTIVE

The objective of this research is to provide design guidance and specifications for widening of existing bridges in seismic regions.  The specific goals of this research are summarized in the following tasks:

- Conduct a literature search to review all relevant research projects;

- Review recent bridge widening projects, challenges encountered, resolutions made, and lessons learned;

- Conduct analytical study for representative bridge widening projects including foundation;

- Develop comprehensive design example for typical bridges;

- Develop design specifications for widening of existing bridges in seismic regions;

- Submit a final report documenting the entire research effort, and recommended revisions to the LRFD Bridge Design Specifications for AASHTO SCOBS consideration.

- URGENCY

Owners, Designers, Contractors and bridge community will have the information they need to make better informed decisions regarding the design, construction, and specifications for bridge widenings in seismic regions that are economically feasible and perform as intended.

- FUNDING REQUESTED AND TIME REQUIRED

It is estimated that this research will take 12 months to complete and will require $50,000.

- CONTACT PERSON

Bijan Khaleghi, Ph.D., P.E., S.E.

State Bridge Design Engineer

Bridge and Structures Office

Washington State DOT, P.O. Box 47340

Olympia, WA 98504-7340

Phone: (360) 705-7181

Email: khalegb@wsdot.wa.gov