AASHTO STANDING COMMITTEE ON RESEARCH

AMERICAN ASSOCIATION OF STATE HIGHWAY AND TRANSPORTATION OFFICIALS

NCHRP Problem Statement

I.	PROBLEM NUMBER

To be assigned by NCHRP staff.

II.	PROBLEM TITLE

Assessment of Small and Medium Sign Supports, Work Zone Devices, and Luminaires to MASH.

III.	RESEARCH PROBLEM STATEMENT

Recent testing of Small and Medium Sign Supports and Work Zone Devices has been problematic with both of the test vehicles required in the 2009 AASHTO Manual for Assessing Safety Hardware (MASH) at both the Texas A&M Transportation Institute (TTI) and the Midwest Roadside Safety Facility (MWRSF) at the University of Nebraska-Lincoln. Many of these devices have previously been successfully full-scale crash tested to NCHRP Report 350 Recommended Procedures for the Safety Performance Evaluation of Highway Features.  Only the small car (1800-lb, Geo Metro or similar) test was performed under NCHRP Report 350.  MASH requires testing with both a small car (2420-lb, Kia Rio or similar) and a pickup truck (5000-lb, ½-ton Dodge Quad Cab or similar) into these types of devices.  Occupant Impact Velocities (OIV’s) and Occupant Ridedown Accelerations (ORAs) have not been a problem because of the increased weight of the test vehicles, even with the commensurate reduction in impact speed in Test 3-60.  However, the change in frontal geometry (i.e. bumper heights, increased frontal area, and wrap around distances) and increased ground clearance has changed the interaction between the vehicle and object struck.  In general, small and medium sign supports used to pass over the top of the impacting vehicle with limited to no vehicle contact.  With the newer MASH vehicles, they are now interacting with the windshield and roof of the current test vehicles and failing the occupant compartment intrusion and/or penetration requirements of MASH.  Similarly, vehicle interactions with portable work-zone devices are causing unacceptable windshield and roof penetrations and/or deformations as well as floor pan penetrations.  No testing has been conducted to date on luminaires (light poles) under MASH, but this recent testing on other break away and portable devices at least raises the question as to the expected performance of breakaway luminaire poles under the MASH impact safety criteria. The addition of objective vehicle intrusion and deformation criteria has also brought into question the future usefulness of pendulum / bogie testing of breakaway supports.

IV.	LITERATURE SEARCH SUMMARY

Most indications of potential problems have been identified with recent testing at MwRSF and TTI.  However, because of the reduced occupant compartment deformations, particularly in the roof crush, older accepted tests under NCHRP Report 230 and NCHRP Report 350 would be failed under MASH.  Under acceptance letter SS08, (http://safety.fhwa.dot.gov/roadway_dept/policy_guide/road_hardware/breakaway/pdf/ss08.pdf), the perforated tube would clearly fail under today’s guidelines when it was previously deemed marginally acceptable.  Similarly, potential previously successful tests under NCHRP Report 230 and 350 on luminaires may be problematic under MASH test conditions.  Subsequent MASH testing with a pickup truck on the perforated steel tube sign supportsunder NCHRP Project 22-14 (3), Bullard, et. al.(TTI-2010) demonstrated unacceptable occupant compartment intrusion at the top of the windshield.  Under TxDOT Project 9-1002-12-5, Dobrovolny, et. al. (TTI-2013) also found unacceptable performance with the 7-ft mounted temporary work-zone sign when impacted at 90 degrees by the MASH pickup due to cutting of the roof sheet metal and subsequent occupant compartment intrusion.  Schmidt, et. al. (MwRSF -2011) published Analysis of Existing Work-Zone Sign Supports Using Manual for Assessing Safety Hardware Safety Performance Criteria http://dx.doi.org/10.1080/19439962.2011.599015.  The testing reviewed in this paper indicated work-zone devices previously tested and accepted under NCHRP Report 350 were subjected to testing with the MASH small car and pickup, four (4) with the car and four (4) with the pickup.  Five (5) of the eight (8) devices failed due to occupant compartment intrusion and/or penetration. Sicking, et. al. (MwRSF-2010) under TRP-03-225-10 published Safety Investigation and Guidance for Work-Zone Devices in Freight Transportation Systems Subjected to Passenger Car and Truck Impacts with New Crash Standards.  This report further documents the results of this work-zone sign support testing and evaluation program using MASH test vehicles.

V.	RESEARCH OBJECTIVE

The objective of this research is to identify currently-accepted small and medium sign supports, work-zone devices, and luminaires under prior crash test standards that either are or will likely be problematic under the current MASH crash test standard.  Likely tasks will include:

Phase I

- Literature review

  - Current Testing on these categories of devices

  - Previous Acceptance Letters that may indicate marginal test outcome

  - Possible Survey of States to gather field data on potential issues.

- Summarize Current State of Knowledge

- Identify Accepted Non-Proprietary devices in these three categories which have increased         propensity for failure under MASH testing

Phase II

- Conduct simulations to further refine potential problem areas

- Conduct full-scale crash tests of products in wide spread use to verify simulation results, up        to 10 tests

- If possible, identify potential retrofits or modifications to breakaway supports and work              zone devices to improve performance.

- Discuss how pendulum and/or bogie vehicle testing can be used to reduce costs of crash             testing under MASH criteria.

The final deliverable will identify known products currently in wide spread use having poor performance and recommend retrofits/modifications for resulting in acceptable safety performance.

VI.	ESTIMATE OF PROBLEM FUNDING AND RESEARCH PERIOD

Recommended Funding:

It’s estimated the funding for Phase I will be $100,000.  Phase II is estimated at $500,000.

Research Period:

Phase I is estimated at six (6) months and Phase II is estimated at eighteen (18) months.

VII.	URGENCY, PAYOFF POTENTIAL, AND IMPLEMENTATION

Phase I is extremely urgent since issues have been identified by both MwRSF and TTI.  Phase I incorporates a Survey that will serve as an In-Service Performance Evaluation of these products and will help assist in the urgency of Phase II.   If the Phase I survey results identifies that there is insufficient field data requiring a Phase II, possible modifications to the MASH requirements on occupant compartment deformation may need to be proposed to AASHTO.

If a Phase II is required, it will greatly assist States in the prioritization of upgrading possibly deficient hardware.  Furthermore, Phase II would likely assist the States with selecting, retrofitting, and modifying those designs to garner improved safety performance.  Possible guidance will also be provided to assist industry with providing similar improved proprietary products used by the various states.

VIII.	PERSON(S) DEVELOPING THE PROBLEM

Dr. Dean C. Alberson, Assistant Agency Director and Research Engineer, TTI,

3135 TAMU, College Station, TX

(979) 458-3874, d-alberson@tamu.edu

Dr. Ron Faller, Director and Research Associate Professor at Midwest Roadside Safety Facility,

130 Whittier Research Center, Lincoln, NE 68583-0853

(402) 472-6864, rfaller@unl.edu

Mr. Nicholas Artimovich, Highway Engineer, FHWA

1200 New Jersey Avenue SE, Room E71-322, Washington, DC  20590

(202) 366-1331, nick.artimovich@dot.gov

Ms. Aurora (Rory) Meza, Director, Roadway Design Section, TxDOT

125 E. 11th Street, Austin, TX  78701-2483

(512) 416-2678, rory.meza@txdot.gov

IX.	PROBLEM MONITOR

Ms. Aurora (Rory) Meza, Director, Roadway Design Section, TxDOT

125 E. 11th Street, Austin, TX  78701-2483

(512) 416-2678, rory.meza@txdot.gov

X.	DATE AND SUBMITTED BY

August 16, 2013

Keith A. Cota, P.E.

Chairman, Technical Committee on Roadside Safety

New Hampshire DOT

7 Hazen Drive, PO Box 483

Concord, NH 03302-0483

Phone: 603-736-8811

Email: kcota@dot.state.nh.us.